Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	COMMUNICATION THEORY AND SYSTEMS			       Time: 3 hours

Subject Code:	EC242		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the band width of AM signal?

2.	State Carson’s rule.

3.	What is the standard frequency of FM receivers?

4.	Differentiate narrow band FM and wide band FM.

5.	What is image frequency?

6.	What is meant by double spotting?

7.	Define Nyquist sampling rate.

8.	What is AGC?

9.	Write the expression for Noise figure.

10.	Define noise temperature.

PART – B (5 x 3 = 15 MARKS)

11.	Explain the need for modulation.

12.	What is the need for the suppression of carriers?

13.	What are the important characteristics of Receivers?

14.	What are the distortions in diode detectors?

15.	How will you calculate noise in FM signals?

PART – C (5 x 15 = 75 MARKS)

16.	a.	With necessary diagrams, explain a basic communication system.   			     		(10)

b.	Differentiate carrier and base band signals.                                                                    	(5)

(OR)

17.	a.	With neat sketches, explain the various types of modulation in detail.                          	(10)

b.	Discuss the need for wireless communication.                                                               	(5)

18.	a.	How will you calculate the total power in AM signal?                                                  	(10)

b.	Compare AM and FM.                                                                                                    	(5)

(OR)

19.	a.	Derive the mathematical representation of FM signal and draw its spectrum.              	(10)

b.	Compare DSB/SC wave and SSB/SC wave. 										(5)

20.	a.	Give the principle of Armstrong method of frequency modulation.                             	(10)

b.	Compare low level and high-level modulators.						      			(5)

(OR)

21.	Explain the following demodulators with suitable diagrams:

a.	Square law detector 	b.	Ratio Detector

[P.T.O]

22.	a.	With a neat diagram, explain Super heterodyne receiver. 			               		(10)

b.	Discuss the merits of SHR over TRF receivers.                                                             	(5)

(OR)

23.	a.	With necessary block diagram, explain FM receiver.                                                     (10)

b.	Briefly explain diversity reception techniques.									(5)

24.	a.	Calculate the output signal to noise ratio in SSB/SC system.                                         (10)

b.	Discuss   thermal and shot noise in communication system.		                             (5)

(OR)

25.	a.	Calculate the output signal to noise ratio in DSB/SC system.                                        	(10)

b.	Compare SNR with respect to AM and FM			                                        		(5)