Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DIGITAL SIGNAL PROCESSING						       Time: 3 hours

Subject Code:	EC245		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the necessary and sufficient condition on the impulse response for stability of an LSI system?

2.	How many multiplications and additions are required to evaluate N point DFT using radix-2 FFT algorithm?

3.	What are the two main advantages of FIR filter?

4.	For what kind of filters, frequency-sampling method of FIR filter design is suitable.

5.	List the properties of Butterworth filter.

6.	What is the advantage of cascade realization?

7.	What are the two kinds of limit cycle behaviours?

8.	What are the methods used to prevent overflow?

9.	When do you use adaptive filter?

10.	What is pipelining?

PART – B (5 x 3 = 15 MARKS)

11.	Find the 4-point DFT of x(n) = {0, 1, 2, 3}.

12.	What is need for employing window for FIR filter design? What are the desirable characteristics of the window function?

13.	What is warping effect? What is its effect on magnitude and phase response?

14.	What is meant by saturation arithmetic? What is its disadvantage?

15.	What are the different buses of TMS320C54X processor and their functions?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Find the linear convolution of x(n)={1, 2, 3, 2, 1} with h(n) = {2, -2, 3, -3} via circular 	convolution.                                                                                                          		(9)

b.	Find the DTFT of x(n) = ( ½ )|n|                                                                                   	(6)

(OR)

17.	a.	Derive decimation-in-time radix-2 FFT algorithm for evaluating N point DFT and draw 	flow graph for 8-point sequence.                                                                              	(7)

b.	Determine the impulse response h(n) for the discrete-time system described by the following difference equation: y(n) - 3y(n-1) – 4y(n-2) = x(n) + 2 x(n-1)				(8)

18.	a.	Design a length-5 FIR band-stop filter with a lower cut-off frequency of 2000Hz, an upper 	cut-off frequency of 2400 Hz, and a sampling rate of 8000Hz using Hamming window 	function. Determine the system function H(z).                                                 		(12)

b.	What is meant by linear phase response? Explain.                                                       	(3)

(OR)

19.	a.	Obtain linear phase realization and direct form realization of an FIR system having system 	function H(z)= (1+0.5z-1z-2)(1+0.25z-1+z-2)                                                          		(5)

b.	Design a length-7 linear phase FIR filter with cut-off frequency of 1500Hz using frequency-sampling method with sampling rate of 10000Hz.           					(10)

[P.T.O]

20.	a.	Convert the following analog filter into equivalent digital filter using impulse invariant 	technique. Assume sampling period Ts = 1 second. H(s)=1(s+1)(s+2)				(7)

b.	Obtain direct form-II and parallel realizations of the system described by the difference equation y(n)+0.1y(n-1)-0.72y(n-2) = 0.7x(n) – 0.252x(n-2)						(8)

(OR)

21.	Design a second order digital Butterworth filter with a cut-off frequency of 3400Hz at a sampling frequency of 8000Hz using bilinear transformation technique. Realize the designed filter in direct form-I and direct-II structures.

22.	a.	Explain what is meant by block floating point representation. What are its advantages?

(6)

b.	Consider a second order IIR filter with system function given by                              	(9)

H(z)=1(1-0.5z-1)(1-0.45z-1).	Find the effect of quantization on pole locations of the above system function in direct form and in cascade realizations. Assume 3-bit representation is used.

(OR)

23.	a.	The output signal of an A/D converter is passed through a first order LSI system having 	difference equation y(n) - 0.85y(n-1) = 0.15x(n).  Find the steady state output noise power 	due to quantization at the input of the filter.                                                 			(6)

b.	Explain the characteristics of the limit cycle oscillation with respect to the system described by the difference equation y(n) = 0.95y(n-1)+x(n) for the input x(n) =0.875(n) with 4-bit number representation and determine the dean band of the filter. 			(9)

24.	a.	Draw the block diagram of basic Wiener filter and derive Wiener-Hopf equations.  	(9)

b.	Explain the basic Harvard architecture with neat block diagram.                                	(6)

(OR)

25.	a.	Explain the basic LMS adaptive algorithm.                                                                	(9)

b.	Explain min, max and squaring instructions of TMS320C5X processor.                  	(6)