Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	TRANSMISSION LINES AND WAVE GUIDES 			       Time: 3 hours

Subject Code:	EC280		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are balanced and unbalanced transmission lines? Give examples.

2.	For a transmission line, if VSWR is 2, compute the percentage of transmitted power and reflected power.

3.	What are the characteristics and applications of   half wave line?

4.	Mention the significance of smith chart.

5.	Evanescent modes in waveguides are always_______. Why?

6.	Define phase velocity and group velocity.

7.	Why TEM mode is not possible in waveguides?

8.	What is meant by dominant mode?

9.	_______mode in a cavity resonator permits widest tuning range and the tuning is accomplished by varying   ________ in it.

10.	The main disadvantage of circular waveguide is that its cross section is larger than that of rectangular waveguide for carrying the same signal. (True / False).

PART – B (5 x 3 = 15 MARKS)

11.	Obtain the relation between reflection coefficient and VSWR.

12.	Explain the merits and applications of Quarter wave transformer.

13.	Show that for a TE10 mode, a frequency of 8 GHz will pass through a wave guide of 	dimensions a = 1.5 cm, b= 1 cm if a dielectric with ε r  = 4 is inserted inside the guide.

14.	Enumerate the characterisics of TEM waves.

15.	Compute the lowest resonant frequency of a rectangular cavity resonator having width 2 cm, height 1cm, length 3 cm.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Describe in detail the different types of transmission lines, their merits and demerits with 	neat diagrams.	      			                   				                             		(10)

b.	A loss less transmission line with Z o = 75 ohms is terminated by a Z L  = 130 ohms. The magnitude of the incident wave is 15 V.  Find the maximum and minimum values of        voltage on the line.									       						(5)

(OR)

17.	a.	Derive the standard transmission line equations.					      				(9)

b.	A 75-ohm line that is half wave long is terminated in a load resistance of 300 ohms. Determine the input impedance. If the same line is then operated at a frequency half of the original operating frequency, then what would be its effect on the input impedance?	(6)

18.	a.	Define and explain:

i)	Standing wave    ii)	Traveling wave    iii)	Flat line     iv)   Commensurate line.(10)

b.	Derive the Z in for a loss less Quarter wave transformer terminated by an impedance ZL.

(5)

(OR)

[P.T.O]

19.	a.	Derive the circle equations of smith chart. Explain the procedure to construct smith chart.																					(8)

b.	Draw a near schematic diagram of a Single Stub Tuner and explain. Compare the features 	of it with double stub tuner.								                   			(7)

20.	Explain the characteristics of TE and TM waves. Derive their characteristics equations.

(OR)

21.	a.	Explain the concept of attenuation of TE and TM waves in parallel plane guides.		(9)

b.	Define and explain the following

a.	Skin depth    	b.	Brewster Angle    	c.	Wave impedance					(6)

22.	a.	Discuss various methods of excitation of modes.									(8)

b.	A standard air filled X band rectangular waveguide has dimensions a= 0.02286m and b =

0.0101m.   Find the fc, λc for the guide. If the signal frequency is 9 GHz, find λg, βg , Zg and 		vp of  TE10 mode.  									      						(7)

(OR)

23.	a.	Derive the characteristic equations of TM waves in a Rectangular waveguides.		(9)

b.	Prove that TM 01 and TM 10 modes do not exist in a rectangular waveguide.                 (6)

24.	a.	Derive the characteristic equations of TE waves in circular waveguides.	                 	(10)

b.	Enumerate and explain the advantages and disadvantages of circular waveguides.      	(5)

(OR)

25.	a.	Explain the characteristics of rectangular and circular cavity resonators. 			(8)

b.	The resonant frequency of a cavity is 8.8 GHz . It is critically coupled to an external circuit.  The measured bandwidth with loading is 4 MHz. Calculate the loaded and unloaded Q’s.																(7)