Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTRON DEVICES									       Time: 3 hours

Subject Code:	EC283		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the force acting on an electron at rest?

2.	Define electrostatic deflection sensitivity of a CRT.

3.	Define drift current.

4.	What is depletion region?

5.	What is the current and voltage gain of a common collector transistor configuration?

6.	Why is a ‘transistor’ so called?

7.	What is the function of the gate in a FET?

8.	What is intrinsic standoff ratio?

9.	What does varactor stand for?

10.	Mention the application of an opto coupler.

PART – B (5 x 3 = 15 MARKS)

11.	What is the effect of force on charged particle in an electric field?

12. 	Write the diode current equation and explain its significance in forward and reverse bias.

13.	What is base width modulation in transistor?

14.	Draw the drain characteristic of FET and explain.

15. 	Explain the principle of operation of LED.

PART – C (5 x 15 = 75 MARKS)

16. 	Describe with a block diagram the main functional units of a cathode ray tube.

(OR)

17. 	Describe with necessary diagram and equation the motion of electron when subjected to combined electric and magnetic fields in parallel and perpendicular.

18.	a.	Explain the electric field in an unbiased PN junction.								(5)

b.	With neat diagram, explain the PN junction diode operation under forward and reverse based conditions.  Draw the VI characteristics.									(10)

(OR)

19. 	Describe with equations, the diffusion capacitance and transition capacitance of a PN junction.

20. 	a.	Draw the structure of an npn transistor and explain its current components.			(7)

b.	Draw the output characteristics of a transistor in CE configuration and explain the various 	regions in the characteristics.													(8)

(OR)

21. 	a.	Draw the Eber Moll model of a transistor and explain its significance.				(10)

b.	Calculate  and  for a given transistor with IC = 5mA, IB = 50A and ICO = 1A.	(5)

[P.T.O]

22. 	 a.	Draw the structure and explain the operation of an n-channel enhancement MOSFET.

(10)

b.	Compare JFET and MOSFET.													(5)

(OR)

23. 	a.	Describe the operation of a SCR using two transistor analogy.						(8)

b.	Draw the structure of a UJT and explain its V-I characteristics.						(7)

24. 	Describe the operation of a tunnel diode and explain the various stages of tunneling.

(OR)

25. 	 Describe the structure, operation and V-I characteristics of a DIAC and TRIAC.