Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DIGITAL ELECTRONICS  								       Time: 3 hours

Subject Code:	EC284		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Convert (24.6)8 to decimal number.

2.	The Gray code of 1101 ______.

3.	Bubbled AND gate is equal to ______.

4.	Define Encoder.

5.	Define rise time.

6.	Draw the truth table of D-flip flop.

7.	Define Johnson counter.

8.	What is dynamic hazard?

9.	Define fan-in.

10.	What is skew?

PART – B (5 x 3 = 15 MARKS)

11.	Convert the (3A.2F)16 to decimal number and (A72E)16 to octal number.

12.	Draw the logic diagram of full adder with truth table.

13.	Compare synchronous and asynchronous sequential circuits.

14.	Compare Moore and Mealy models.

15.	Mention the classification of saturated bipolar logic families.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Illustrate the following codes with an example

i)	Excess –3-code	ii)	BCD code	iii)	ASCII code							(3x3)

b.	State Demorgan’s theorem and apply Demorgan’s theorem to the following expression     	Y = (A+B+C)D.  																(6)

(OR)

17.	a.	Define minterm and maxterm with example.					 					(7)

b.	Simplify the expression using Karnaugh map Y= π(0,1,4,5,6,8,9,12,13,14).    		(8)

18.	a.	Realize the following logic gates using NOR gates

i)	NAND		ii)	AND	iii)	OR											(3x3)

b.	Design a 4-bit binary adder using combinatorial logic.							(6)

(OR)

19.	a.	Define Multiplexer. Explain in detail about  4 to 1 multiplexer with neat circuit diagram 	and truth table.																(7)

b.	Design a binary to BCD code converter using unused combinations as don’t care conditions. 																	(8)

[P.T.O]

20.	Draw and explain the logic diagrams of JK and T flip-flops. Also, state their characteristics and truth tables.

(OR)

21.	a.	Design Mod 4 synchronous counter using JK flip flop and implement it.			(10)

b.	Give the comparison between synchronous and asynchronous counters.				(5)

22.	Explain Static, dynamic and essential hazards in digital circuit. Give hazard – free realization for the following Boolean function f(A,B,C,D) = m(2,3,5,7,10,14).

(OR)

23.	Explain the sequential logic circuit partitioning with one example. Also explain the rules for partitioning.

24.	Draw a TTL –NAND gate with totem pole output and explain its working. Mention its advantages and disadvantages.

(OR)

25.	a.	Draw the basic CMOS inverter and NOR gates and explain their operations.   		(10)

b.	Explain the Integrated Injection Logic (IIL) with neat logic diagram.	        	 	(5)