Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	SOLID STATE CIRCUITS								       Time: 3 hours

Subject Code:	EC288		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The ripple factor for the half wave rectifier is ________.

2.	What is shunt voltage regulator?

3.	What is the drawback of fixed bias using BJT?

4.	The intersection of DC load line and the output characteristics of a transistor are called ________.

5.	If G1, G2 and G3 are the individual voltage gains of a three stage amplifier, then total voltage gain G = ________.

6.	Which configuration eliminates crossover distortion?

7.	How much percentage feedback is present in an emitter follower amplifier?

8.	What is the CMRR of an ideal differential amplifier?

9.	State Barkhausen criterion.

10.	What is Stagger-tuning?

PART – B (5 x 3 = 15 MARKS)

11.	A full wave rectifier uses an LC filter with L=20H and C=15µF. The applied voltage is v=300sin 2 π×50t. Find the ripple factor.

12.	Draw the circuit diagram of fixed-bias using BJT and find IB and VCE.

13.	Calculate the effective resistance seen looking into the primary of a 15:1 transformer connected to a 10Ω load.

14.	What are the advantages of negative feedback amplifier?

15.	Describe the operation of Crystal Oscillator with neat circuit diagram.

PART – C (5 x 15 = 75 MARKS)

16.	a.	With a circuit diagram and necessary waveforms describe the operation of a full wave 	rectifier.			       															(7)

b.	In a full wave rectifier the load resistance RL=2KΩ and diode forward resistance Rf =400Ω. The voltage applied to each diode is 240 sin 50t. Find i) Peak value of current ii) Average value of current iii) RMS value of current iv) Rectifier efficiency.            		    	(8)

(OR)

17.	a.	With necessary diagram, derive the expression for ripple factor of full wave rectifier with 	Capacitor filter.  		              													(8)

b.	Draw the circuit diagram of transistorized series feedback voltage regulator and describe 	its working. 															    		(7)

18.	Draw the circuit diagram of Self-bias using BJT and derive an expression for its stability factor.

(OR)

19.	In a Voltage divider bias circuit using FET, calculate VG, ID, VDS, VGS and VS using mathematical approach. Given VDD=12V, RD=1.2 kΩ, RS= 2kΩ, R1=20kΩ, R2=10kΩ IDSS=12mA and VP= -4V.

[P.T.O]

20.	Explain the operation of transformer coupled amplifier with neat circuit diagram and derive its mid frequency gain of an amplifier.

(OR)

21.	With neat sketch, explain the operation of Class-B power amplifier and derive its maximum efficiency.

22.	With a neat block diagram explain about voltage shunt feedback and voltage series feedback and derive its voltage gain, input impedance and output impedance.

(OR)

23.	Describe the operation of differential amplifier using BJT. Also derive the expression for double-ended voltage gain and draw its ac equivalent circuit.

24.	With neat circuit diagram explain the operation of Hartley oscillator and derive its frequency of oscillation.

(OR)

25.	With neat sketch explain the operation of a single tuned amplifier and explain its frequency response.