Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	PULSE AND WAVE SHAPING CIRCUITS				       Time: 3 hours

Subject Code:	EC289		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Why are RC circuits commonly used compared to RL circuits?

2.	Why is the capacitor in an RC high-pass circuit called a blocking capacitor?

3.	What are the applications of a bistable multivibrator?

4.	How can hysteresis be eliminated in a Schmitt trigger?

5.	What is blocked condition in an astable multivibrator?

6.	What type of triggering is used in a monostable multivibrator?

7.	Why are time-base generators called a sweep generators?

8.	Define slope error.

9.	What is a sampling gate?

10.	How are pulse width and pulse repetition rate controlled in blocking oscillator?

PART – B (5 x 3 = 15 MARKS)

11.	What do you mean by clamping? What the other names of a clamping circuit?

12.	Compare the saturated and non-saturated binary.

13.	How are linearly varying current waveforms generated?

14.	What are the principles common to Miller and Bootstrap sweep generators?

15.	How can pedestal be reduced in sampling gates?

PART – C (5 x 15 = 75 MARKS)

16.	Derive an expression for the percentage tilt of the output of a high pass RC circuit with large time constant excited by a symmetrical square wave with zero average value.

(OR)

17.	With the help of a neat circuit diagram, explain the working of an emitter-coupled clipper.

18.	With the help of a neat circuit diagram, explain the working of fixed bias Bistable multivibrator. Derive the expression for resolution time and switching speed of the multivibrator.

(OR)

19.	Draw the circuit of a Schmitt trigger and explain its working. Derive the expression for LTP and UTP. Also explain how a Schmitt trigger can be used as squaring circuit and amplitude comparator.

20.	With the help of neat circuit diagram, explain the working of a collector coupled Monostable multivibrator. Also, derive an expression for the gate width of monostable multivibrator.

(OR)

21.	With the help of a circuit diagram, explain the operation of emitter coupled astable mutivibrator and discuss its advantages and disadvantages.

[P.T.O]

22.	With the help of a neat circuit diagram, explain the working of a transistor constant current sweep circuit.

(OR)

23.	With the help of a neat circuit diagram and waveforms, explain the working of a transistor bootstrap time base generator.

24.	Explain the circuit of a unidirectional sampling gate. Identify the cases when input signal is lost during transmission and when a pedestal appears in the output waveform.

(OR)

25.	How do sampling gates differ from logic gates? Draw the circuit diagram of unidirectional sampling gate and explain its working.