Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	BASIC ELECTRICAL ENGINEERING 					      Time: 3 hours

Subject Code:	EE101		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Temperature coefficient of Resistance is defined as _________.

2.	How many parallel paths are there in a lap wound machine?

3.	Transformer is a static machine. (True/False)

4.	Expression for the Alternating voltage is _________.

5.	List the two types of DC distribution system.

6.	Mention one advantage of RCC pole type of Line support.

7.	Slip = _________.

8.	Speed equation of dc motor is _________.

9.	_________ and _________ coils are available in Wattmeter.

10.	DPIC means _________.

PART – B (5 x 3 = 15 MARKS)

11.	Derive the equation for the emf induced in dc generator.

12.	Define the following terms: Frequency, Form factor, Peak factor.

13.	Mention the advantages of underground cables.

14.	Write the applications of Split phase, Capacitor start and Capacitor start capacitor run induction Motor.

15.	Briefly explain about earthing.

PART – C (5 x 15 = 75 MARKS)

16.	Draw and explain the Open circuit characteristics, Internal and External characteristics of DC shunt generator.

(OR)

17.	a.	Three resistances R1, R2 and R3 are connected in series. Derive an expression for the equivalent 	resistance.																	(3)

b.	A 8 pole lap connected generator has a useful flux/pole of 0.05 wb. If the induced voltage at 400 	rpm is 300 V, find the number of armature conductors.								(3)

c.	With necessary diagram, explain about compound generator. Write the voltage and current equations.   																	(9)

18.	a.	Describe the constructional details of a transformer.									(7)

b.	The equation of an alternating current is given by i=40 sin (314t). Determine

(i) Maximum value of current  (ii) Average value of current (iii) RMS value of current

(iv) Frequency and angular frequency (v) Form factor (vi) Peak factor 					(8)

(OR)

19.	a. 	Derive the expression for RMS, average value, form factor and peak factor of sinusoidal 	quantity. 																		(9)

b.	Write about the different types of transformers.										(6)

20.	a.	Write short notes on the following: 												(5+5)

i)	Radial distribution system			ii)	Ring main system.

b.	 Explain about conductors used in transmission system.   								(5)

(OR)

[P.T.O]

21.	a.	Brief about the different types of transmission lines. 									(5)

b.	With a neat block diagram explain the working of a thermal power plant.  				(10)

22.	a.	Explain the principle of operation of dc motor. 										(10)

b.	Derive the expression for frequency of rotor current a three phase induction motor.  		(5)

(OR)

23.	a.	Explain the construction of a three phase induction motor.  						 	(10)

b.	A three phase induction motor runs almost at 1000 rpm on no load and 960 rpm on full 	load when 	supplied with 50 Hz. Calculate (i)	    number of poles 	(ii)  slip at full load    (iii) Frequency of rotor 	(iv) 	speed of the motor at 8 percent slip.					 						(5)

24.	a.	With the help of neat supporting diagrams, explain the construction and working of a Induction 	type energy meter. 													 			(8)

b.	Describe the operation of staircase wiring with neat diagram. 						 	(7)

(OR)

25.	a.	Write notes on wiring materials and accessories used in domestic wiring.			 		(7)

b.	With suitable diagram, explain the working of a Moving iron Instrument.			 		(8)