Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTRIC CIRCUIT ANALYSIS						       Time: 3 hours

Subject Code:	EE201		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are dependent sources?

2.	Write the voltage division rule to find the voltage across the resistance R3, when three resistances R1,R2and R3 are connected in series.

3.	Write the conditions for an AC circuit to behave like a resonant circuit.

4.	At what condition, the two wattmeters read equal and opposite values during the two-wattmeter method of power measurement?

5.	What is an ideal transformer?

6.	Write the relation between self inductances, mutual inductance and coefficient of coupling.

7.	Draw the Thevenin’s equivalent circuit.

8.	Write the condition for maximum power transferred in AC circuits.

9.	What is meant by forced response?

10.	Write the condition for under damping in RLC series circuit.

PART – B (5 x 3 = 15 MARKS)

11.	Obtain the single current source between the terminals of the circuit shown in the following figure.

12.	A coil having a resistance of 7 ohms and an inductance of 31.8 mH is connected to 230 volts, 50Hz supply.  Calculate the current flowing through the coil.

13.	Obtain the equivalent inductance when two coils are inductively connected in series aiding.

14.	State and explain Millman’s theorem.

15.	Define time constant in RL series circuit and write the significance of time constant.

PART – C (5 x 15 = 75 MARKS)

16.	Calculate the current in all the resistors of the network shown in figure below, using Kirchhoff’s law.

(OR)

[P.T.O]

17.	a.	Find the equivalent resistance in between terminals A and B of the circuit shown in figure 		below.                                                                                							(7)

b.	Find the currents flowing through all the elements of the circuit shown in figure below.

(8)

18.	A series RLC circuit consists of a resistance 20Ω, inductance 0.2H, and capacitance of 200μF. A supply of 230V with 50Hz is given to a series circuit. Find the total impedance, current, power, power factor, voltage across coil, and voltage across the capacitance.

(OR)

19.	Three impedances Z1 = 20∠300Ω, Z2 = 40∠600 and Z3 =10∠−900 are delta connected to a 400V, 3 phase system.  Determine the (i) phase currents (ii) line currents and (iii) total power consumed by the load.

20.	a.	From the first principles, explain the matrix method of analyzing a circuit.  			(6)

b.	For the circuit shown in figure below, determine the voltages at nodes 1 and 2 with respect 	to the datum.                                                                             						(9)

(OR)

21.	Determine the node voltages and the current through the resistors using mesh method for the network shown in figure below.

22.	Verify Reciprocity theorem for the circuit shown in figure below.

(OR)

[P.T.O]

23.	Determine the current flowing through the 5Ω resistor across the terminals AB in the circuit shown in figure below by using Norton’s theorem.

24.	A series RL circuit with R = 30 ohm and L = 15H has a constant voltage V=60V applied at t=0. Determine current i, the voltage across resistor and the voltage across the inductor.

(OR)

25.	Obtain the sinusoidal response of a RC network shown in figure below.