Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTRICAL TECHNOLOGY							       Time: 3 hours

Subject Code:	EE205		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Cast iron is replaced by _______ for the construction of a DC machine yoke.

2.	A commutator converts alternating voltage to _______.

3.	A transformer is a static _______ device for the transformation of the alternating current system into another one with the same frequency with varying other characteristics in particular, current or voltage.

4.	Power transformers should be designed to have _______ efficiency at or near full load.

5.	An exciter is nothing but a _______.

6.	The speed of a p-pole synchronous in rpm is given by _______.

7.	Relative motion between the primary and secondary structures is produced by the electromagnetic force corresponding to the power thus transferred across the _______ by induction.

8.	The shaft of an induction motor is short and stiff. A small deflection in case of rotor may produce large irregularities in the air gap leading to the production of unbalanced _______.

9.	The torque exerted by the rotor magnetic field of a PM stepping motor with unexcited stator is called _______ torque.

10.	A DC servomotor is similar to a regular dc motor except that its design is modified to cope with _______.

PART – B (5 x 3 = 15 MARKS)

11.	List the types of DC motors and mention their applications.

12. 	Mention different methods of transformer cooling.

13.	What do you mean by transformer efficiency?  How does it differ from all day efficiency?

14.	List some advantages of a three phase induction motor.

15. 	List some applications of stepper motor.

PART – C (5 x 15 = 75 MARKS)

16. 	A 440V, 4 pole, lap connected shunt motor has a no load input current of 15A and a shunt field current of 10A.  At full load, it takes a current of 150A.  If armature resistance is 0.1 ohm, flux per pole on no load is 0.05 Weber, number of armature conductors is 750 and contact drop per brush is 1V.  Calculate (a) no load speed (b) full load speed (c) speed regulation.

(OR)

17. 	Explain the construction and principle of operation of DC motor.  Also derive the expression for torque developed in a DC series motor.

[P.T.O]

18.	a.	Derive the EMF equation for a single-phase transformer.							(8)

b.	A single-phase transformer is connected to a 230V, 50Hz supply.  The net cross sectional are of the core is 60cm2.  The number of turns in primary is 500 and in the secondary 100.  Determine (i) transformation ratio (ii) EMF induced in the secondary winding

(iii) maximum value of flux density in the core.									(7)

(OR)

19. 	A 15kVA, 2000/200V transformer has an iron loss of 250W and full load copper loss of 350W.  During the day it is loaded as follows:

| No. of hours | Load | Power factor |
| --- | --- | --- |
| 9 | ¼ load | 0.6 |
| 7 | Full load | 0.8 |
| 6 | ¾ load | 1.0 |
| 2 | No load | - |

20. 	A 3-phase 16-pole, star connected alternator has 192 stator slots with 8 conductors/slot and the conductors of each phase are connected in series.  The coil span is 150 electrical degrees.  Determine the phase and line voltage if the machine runs at 375 rpm and the flux per pole is 64mWb distributed sinusoidally over pole.

(OR)

21. 	Explain the constructional features and operating principle of three-phase alternator?  Also derive the expression for frequency of induced emf in the alternator.

22. 	 Explain in detail about the starting methods of induction motors with suitable diagram.

(OR)

23. 	A three-phase slip ring induction motor gives a reading of 60V across slip rings on open circuit when at rest with normal stator voltage applied.  The motor is star connected and has impedance of (0.6 +0.6j)  per phase.  Find the rotor current when the machine is (a) at standstill with slip rings joined to a star connected starter with a phase impedance of (5+j4); and (b) running normally with 4 percent slip.

24. 	 Explain in detail about operation and construction of AC servomotors.

(OR)

25. 	 Explain with neat sketch the construction and working of a single-phase capacitor start capacitor run induction motor.