Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTRIC CIRCUITS									       Time: 3 hours

Subject Code:	EE245		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	State ohm’s law.

2.	Name the two types of independent sources.

3.	Define ‘form factor’ of an alternating quantity.

4.	Draw the voltage – current phasor relationship of pure resistive element.

5.	Mesh analysis is based on Kirchhoff’s ________ law.

6.	What are the two methods of analyzing circuits?

7.	Draw the Norton’s equivalent circuit.

8.	State Thevenin’s theorem.

9.	Define ‘selectivity’ of a resonant circuit.

10.	What do you mean by bandwidth of a resonant circuit?

PART – B (5 x 3 = 15 MARKS)

11.	Convert the given star in the figure into an equivalent delta.

12.	Define power factor in terms of impedance and power components.

13.	Write the mesh current equations in the circuit shown in the figure.

14.	State and prove maximum power transfer theorem.

15.	Derive the expression for resonant frequency.

PART – C (5 x 15 = 75 MARKS)

16.	Determine the equivalent resistance between the terminal A and B for the circuit shown in the figure.

10Ω

(OR)

[P.T.O]

17.	A Wheatstone bridge consists of the following resistances. Branches AB = 3 Ω, BC = 6 Ω, CD = 12 Ω and DA = 10 Ω. A 2 V cell is connected between points B and D and Galvanometer of resistance 20 Ω is connected between A and C. Find the current through galvanometer by Kirchhoff’s laws.

18.	a.	Obtain the form factor and peak factor of a full – wave rectified sine wave shown in the 	figure.																		(7)

b.	Given two sources of emf V1 (t) = 200 sin ωt and V2 (t) = 200 sin (ωt + π6) are in series. Obtain the resultant emf with reference to the voltage 200 sin ωt. Obtain the rms current and power supplied to a (8 + j 6) Ω impedance.	 								(8)

(OR)

19.	a.	Determine the power factor, true power, reactive power and apparent power in the circuit 			in the figure.																	(7)

b.	A sine wave of V (t) = 200 sin 50t is applied to a 10 Ω resistor in series with a coil. The reading of a voltmeter across the resistor is 120 V and across the coil, 75 V. Calculate the power and reactive volt – amperes in the coil and the power factor of the circuit.		(8)

20.	Find the voltage across the capacitor for the circuit shown in the figure. Using mesh analysis.

(OR)

21.	Find VBD by nodal analysis for the circuit shown in the figure.

[P.T.O]

22.	Calculate the current I through the 160 Ω resistor using Norton’s theorem as shown in the figure.

(OR)

23.	Find the load impedance ZL such that the maximum power can be transferred to the circuit shown in figure.

24.	Obtain the value of XL for which unity power factor parallel resonance occurs in the circuit shown in the figure.

(OR)

25.	For the circuit shown in the figure determine the frequency at which the circuit resonance. Also find the voltage across the inductor at resonance and the Q – factor of the circuit.