Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTRO MAGNETIC FIELDS						       Time: 3 hours

Subject Code:	EE248		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Convert given (5, 3, 2) in Cartesian in to cylindrical coordinates.

2.	What is divergence of curl of any vector?

3.	Write the Laplace’s equation.

4.	Define capacitance.

5.	Define the current density J.

6.	Differentiate between solenoid and toroid.

7.	Define displacement current.

8.	Express the eddy current loss in transformer.

9.	What is intrinsic impedance?

10.	Write Snell’s law.

PART – B (5 x 3 = 15 MARKS)

11.	Write the surface integral in Cartesian and cylindrical coordinates.

12.	Define gauss’ law and mention its application.

13.	Define self and mutual inductances.

14.	Write the Maxwell equations in point form.

15.	Define the skin depth.

PART – C (5 x 15 = 75 MARKS)

16. 	Derive the divergence theorem with examples.

(OR)

17.	a.	Find the divergent of A  at (2,-1,3)  when A= 2xy ax + Z aY + YZ aZ             	 		(7)

b.	Find gradient of (2xy+ 3z) at (1,1,1)                                                          			(8)

18.	Four charges of 1C placed at corners of a square with 1m side. Find the force on any one corner due to all other charges act on it.

(OR)

19.	A volumetric spherical charge of ρ C/m3 with radius a uniformly distributed. Use Gauss law to obtain electric field a) inside b) surface c) outside of sphere 							(3x5)

20.	Find the magnetic field B due to infinite conductor carrying current I at a distance R above the conductor in a free space.

(OR)

21.	Obtain the boundary conditions at interface of two magnetic materials having permeabilities μ1 and μ2.

22.	Write the Maxwell equations in integral and differential form.  How is the Ampere’s law corrected by Maxwell.

(OR)

[P.T.O]

23.	Derive the self-inductance of a solenoid having length L, radius a, having N number of turns carrying the current I.

24.	Derive the electromagnetic wave equation in forward and backward motion.

(OR)

25.	Derive the propagation constant and intrinsic impedance of a loss less dielectric.