Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DC MACHINES AND TRANSFORMERS				       Time: 3 hours

Subject Code:	EE250		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are the basic things required for a generating action?

2.	State the functions of commutator in a d.c. machine.

3.	Write the torque equation of a d.c. motor.

4.	How can the direction of rotation of a d.c. motor be changed?

5.	Can d.c. supply be used for transformers?

6.	Define transformer.

7.	Write the steinmetz empirical formula for hysteresis loss.

8.	Write the two methods of obtaining efficiency and regulation of a transformer.

9.	What is the purpose of instrument transformers?

10.	Write down the choice of transformer connection is used for step up transformer in power system application.

PART – B (5 x 3 = 15 MARKS)

11.	List the main parts of d.c. machine.

12.	Why are series motors never started on no load?

13.	Why are transformers rated in kVA instead of kW?

14.	What are the parameters to be obtained in o.c. and s.c. test?

15.	Draw star-delta connection and obtain line voltage and phase voltage relationship.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the emf equation of d.c. generator.										(6)

b.	A dc series generator has its armature resistance of 0.5 Ω and series field resistance of 0.03 Ω. It drives a load of 50 A. If it has 6 turns / coil and total 540 coils on the armature and is driven at 1500 r.p.m., calculate the terminal voltage at the load. Assume 4 poles, lap type winding, flux per pole as 2 mWb and total brush drop as 2V.						 (9)

(OR)

17.	Draw and explain the load characteristics of dc shunt – series and compound generator end explain its applications.

18.	a.	Explain the principle of operation of dc motor.									(10)

b.	Explain significance of back emf.												(5)

(OR)

19.	a.	Derive the torque equation of a d.c motor.										(8)

b.	A 4 pole, 250 V, d.c. series motor has a wave connected armature with 200 connectors. The flux per pole is 25 mWb when motor is drawing 60 A from the supply. Armature resistance is 0.15 Ω while series field winding resistance is 0.2 Ω. Calculate the speed under this condition.  															(7)

[P.T.O]

20.	a.	Derive the e.m.f. equation of transformer.										(8)

b.	A single-phase transformer has 350 primary and 1050 secondary turns. The primary is connected to 400 V, 50 Hz a.c. supply. If the net cross sectional area of the core is 50cm2 calculate i) The maximum value of the flux density in the core.  ii) The induced e.m.f in the secondary winding. 			 											(7)

(OR)

21.	Derive the equivalent circuit of transformer and draw its approximate equivalent circuit.

22.	Explain the various losses in the transformers and derive the condition for maximum efficiency.

(OR)

23.	Explain how equivalent circuit parameters are determined from o.c and s.c. test readings.

24.	Explain 3Ф transformer connection methods.

(OR)

25.	Explain working principle and connection diagram of potential and current transformers.