Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	INDUCTION AND SYNCHRONOUS MACHINES		       Time: 3 hours

Subject Code:	EE251		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Compare squirrel cage rotor and slip ring rotor.

2.	What is the condition for maximum starting torque of an induction motor?

3.	Name the theory used to explain the principle of operation of single-phase induction motors.

4.	State some applications of linear induction motors.

5.	Define pitch factor.

6.	Which method of predetermination of regulation is called pessimistic method?

7.	What are the special features of synchronous motor?

8.	Define Hunting.

9.	What is meant by regulation?

10.	Define direct axis reactance.

PART – B (5 x 3 = 15 MARKS)

11.	A 3 phase Induction motor is wound for 4 poles and is supplied from 50 Hz system.  Calculate (a) Synchronous speed.  (b) The speed of the motor when slip is 4 %.

12.	Whether the single phase induction motor is self starting or not?  Justify.

13.	What are the conditions to be fulfilled for parallel operation of synchronous generators?

14.	What is the main function of damper winding in synchronous motors?

15.	Write the expression for power and power angle relation of synchronous machines.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the torque equation of induction motor and also draw the Torque–Slip 	characteristics of the same.                                                                  				(8)

b.	The power input to a 3-phase induction motor is 40 kW.  The stator total loss is 1 kW and the mechanical losses are 1.6 kW.  Determine (i) Shaft power.  (ii) Rotor Cu loss.  (iii) Efficiency of the motor with a slip of 3.5 %. 										(7)

(OR)

17.	a.	Explain how the rotor rotates in three-phase induction motor.            				(5)

b.	A 3 phase induction motor has a star connected rotor.  The rotor emf (between slip rings) at standstill is 50 V.  The rotor resistance and standstill reactance are 0.5 Ώ and 3 Ώ respectively.  Find

i)	Rotor current per phase at starting and the slip rings short-circuited.

ii)	Rotor current per phase at starting if a star connected rheostat of resistance 6 Ώ per 	phase is connected across the slip rings.

iii)	Full load rotor current and rotor power factor if the full load slip is 0.04.

iv)	Rotor emf per phase under full load condition.                          					(10)

18.	Explain the construction and operation of single phase Induction Motor.

(OR)

[P.T.O]

19.	Explain using suitable diagrams: 													(3x5)

a.	Split-phase induction motor		b.	Capacitor-start induction motor

c.	Deduce the equivalent circuit of single-phase Induction motor.

20.	a.	Derive the induced emf equation of an alternator.                               				(6)

b.	A 3 phase, 16 pole alternator has the following data.  Number of slots = 192, 	Conductors/slot = 8 (conductors of each phase are connected in series), Coil span = 150 	electrical degrees, Speed of the alternator = 375 rpm and Flux per pole = 50 mWb.  	Calculate the phase and line voltages.                    								(9)

(OR)

21.	a.	Explain the emf and mmf method of an alternator.                            				(10)

b.	A 500 V, 50 KVA single phase alternator has an effective resistance of 0.2 Ώ.  A field current of 10 A produces an armature current of 200 A on short circuit and an emf of 450 V on open circuit.  Calculate the full load regulation at 0.8 lagging PF.                      	(5)

22.	Explain the principle of operation and methods of starting of Synchronous motors.

(OR)

23.	Explain how V and inverter V curves are drawn, using suitable experimental setup.

24.	Explain two-reaction theory analysis of salient pole machine using phasor diagram.

(OR)

25.	Discuss the determination of Xd, Xq and regulation using slip test.