Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ELECTRICAL MACHINE DESIGN						       Time: 3 hours

Subject Code:	EE252		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What are the factors should be considered when selecting the number of armature slots?

2.	Define specific magnetic loading.

3.	Write the Pitchelmayer’s equation.

4.	What are the advantages of higher number of poles in the number of poles selection procedure in DC machine?

5.	What are the types of high voltage windings in the winding selection of a transformer?

6.	What is meant by Circumscribing circle?

7.	How will you improve the power factor in an induction motor when considering air gap length?

8.	How will you decrease the additional copper loss in an induction machine?

9.	Define short circuit ratio of the synchronous machine.

10.	What are the various losses in synchronous machine?

PART – B (5 x 3 = 15 MARKS)

11.	List out the limitations of electrical machine design?

12.	List out guiding factors for choice of no. of armature slots.

13.	A 3 phase induction motor has 54 stator slots with 8 conductors per slot and 72 rotor slots with 4 conductors per slot. Find the No. of stator and rotor turns.

14.	Compare Single layer winding and double layer winding in synchronous machine.

15.	What are the important features to be considered for the selection of armature slots in an synchronous machine?

PART – C (5 x 15 = 75 MARKS)

16.	Discuss how the specific electric and magnetic loadings will be chosen for rotating machine?

(OR)

17.	The armature of a D.C. machine has a diameter of 0.2m, 29 parallel slots each 6mm wide and 25mm deep: the core is 0.15m long with 1 duct 10mm wide, and the air gap 3mm long. Insulation on the stampings is 10% of the thickness.

If the maximum flux density under the pole is 0.9Wb/ m2, determine the mmf required to overcome the reluctance of gap and teeth. Carter’s co-efficient for the slot is 0.275 and for the ducts is 0.39. The magnetization curve for the iron is as follows.

| B (Wb/ m2) | 1.4 | 1.6 | 1.8 | 2.0 | 2.2 | 2.3 |
| --- | --- | --- | --- | --- | --- | --- |
| H( A/m) | 1800 | 3000 | 6500 | 19400 | 63000 | 112000 |

18.	a.	Derive the output equation of a D.C. machine and point out its salient features. 		(8)

b.	State and explain the factors, which govern the choice of specific magnetic loading in a 	D.C. machine.																(7)

(OR)

[P.T.O]

19.	A rectangular field coil is to produce an mmf of 7500 A when dissipating 220W at a temperature of 60˚ C. The inner dimensions of coil are : length = 0.24m, width = 0.10m and height=0.15m. The heat dissipation is 30 W/m2 -˚C from the outer surface, neglecting top and bottom surfaces of coil. Temperature of ambient air is 20˚C. Calculate the thickness of the coil, the space factor and the current density. Resistivity is 0.02Ω/m and mm2.

20.	Determine the main dimensions of the core, the number of turns and the cross – section of the conductors for a 5kVA, 11000/400 V, 50Hz, single phase core type distribution transformer. The net conductor area in the window is 0.6 times the net cross section of iron in the core. Assume a square cross-section for the core, a flux density 1 Wb/ m2 , a current density 1.4 A/ mm2 , and a window space factor 0.2. The height of window is 3 times its width.

(OR)

21.	A1000kVA, 6600/440V, 50 Hz, 3 Phase, Delta/star, core type, oil immersed natural cooled (ON) transformer. The design data of the transformer is: Distance between center of adjacent limbs = 0.47m, outer diameter of high voltage winding = 0.44m, height of frame = 1.24m. core loss = 3.7kw,and I2R loss = 10.5kW.Design the suitable tank for the transformer. The average temperature rise of oil should not exceed 35˚C.

The specific heat dissipation from the tank walls is 6 W/m2 -˚C and 6.5 W/m2 -˚C. due to radiation and convection respectively. Assume that the convection is improved by 35% due to convection.

22.	A 15kW, 400V, 3phase, 50 Hz, 6 Pole Induction motor has a diameter of 3mm and the length of the core 0.12m. The number of stator slots is 72 with 20 conductors /slot. The stator is delta connected. Calculate the magnetizing current per phase if the length of air gap is 0.55m. The gap contraction factor is 1.2. Assume the mmf required for the iron parts to be 35 percent of the air gap mmf. Coil span = 11 slots.

(OR)

23.	A 3 phase 2pole, 50Hz squirrel cage induction motor has rotor diameter 0.20m and core length 0.12m. The peak density in the air gap is 0.55 Wb/m2. The rotor has 33 bars, each of resistance 125 μΩ and a leakage inductance 2 μH. The slip is 6%.  Calculate (i) the peak value of current in each bar (ii) rotor I2R loss (iii) rotor output and (iv) torque exerted. Neglect the resistance of end rings.

24.	A1000 kVA, 3300V, 50Hz, 300rpm, 3phase alternator has 180 slots with 5 conductors per slot. Single layer winding with full pitch coil is used. The winding is star connected with one circuit per phase. Determine the specific electric and specific magnetic loadings, if the stator core is 2.0m and the core length is 0.4m. Using the same loadings, determine the corresponding data for a 1250 kVA, 330V, 50 Hz, 250 rpm, 3 phase star connected alternator having 2 circuits per phase. The machines have 60◦ phase spread.

(OR)

25.	Estimate the diameter, core length, size and number of conductors, number of slots for stator of a 15 MVA, 11kVA, 50Hz, 2Pole star connected turbo alternator with 60◦ phase spread. Assume: Bav= 0.55 Wb/m2 ; ac = 36,000A/m ; current density = 5A/mm2  , peripheral speed = 160m/s.  The winding should be arranged to eliminate 5th harmonic.