Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DIGITAL ELECTRONICS								       Time: 3 hours

Subject Code:	EE257		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Convert (0.503)10 to octal.

2.	Simplify the following expression x (x’ + y).

3.	What is a logic gate?

4.	What do you mean by Fan-out in logic IC families?

5.	What is a priority encoder?

6.	Define Propagation delay.

7.	What do you mean by shift register?

8.	Define state table.

9.	What is the keyword used before signal declaration in VHDL?

10.	Define PAL.

PART – B (5 x 3 = 15 MARKS)

11.	Perform the subtraction X – Y of two binary numbers X = 1010100 and Y = 1000011 using 2’s complements.

12.	Convert the following expression to its max-term canonical form.

f (a, b, c) = a +  ac( b + c)

13.	Design the 2-input NAND gate using the 2:1 MUX.

14.	Draw the block diagram of the Mealy machine and write how Mealy machine differs from Moore machine.

15.	Compare the PROM, PLA, and PAL.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Design a combinational circuit for converting excess-3 to BCD code and implement 	through logic gates.															(10)

b.	Discuss how error detection is done through Hamming codes.						(5)

(OR)

17.	a.	Express the Boolean function F = xy + x’z as a product of max-terms.				(8)

b.	List the basic theorems and postulates of the Boolean algebra.						(7)

18.	a.	Explain in detail about tri-state gate of a digital IC.								(7)

b.	Construct the logic circuit for the following expression using CMOS transistor.

Y = AB + B’C																(8)

(OR)

19.	a.	Discuss in detail about the TTL logic gate configured in open collector and Totem pole 	output.																		(10)

b.	Explain How TTL is interfaced with CMOS.										(5)

[P.T.O]

20.	a.	Design a 4-bit carry look-ahead adder and list out the advantages and disadvantages over 	the ripple carry adder.															(8)

b.	Simplify the following Boolean function using K-map. F(w,x,y,z) =∑ (1,4,,6,12,14,15)

(7)

(OR)

21.	Find all the prime implicants of the function f(a,b,c,d) = ∑m(7,9,12,13,14,15) + dc(4,11) using Quine- McCluskey algorithm.

22.	a.	Let a 4-bit serial in- serial out unidirectional shift register configured using positive edge 	triggered D flip-flops initially contain 1011. Give the contents of the register after each 	positive edge of the clock if the data on the serial input line is 1, 1, 0, 1, 0, 1, 0.    	(5)

b.	Design a sequential circuit with two D flip-flops A and B, and one input ‘x’. When    x = 0, 	the state of the circuit remains the same. When x = 1, the circuit goes through the state 	transitions from 00 to 01 to 11 to 10 back to 00, and repeats.						(10)

(OR)

23.	For the following state diagram, design the sequential circuit using JK flip-flops.

24.	a.	Implement the following functions using PROM PLD.							(7)

F1(a,b,c) =∑m (2,3,4,6)		F2(a,b,c) =∑m (0,1,5,7)

b.	Write a VHDL code of a two - digit decimal counter with entity declaration and 	architecture body.															(8)

(OR)

25.	a.	Illustrate how a PLA can be used for combinational logic design with reference to the 	functions:  f1(a,b,c) = m(0,1,3,5); f2 (a,b,c) = m(0,2,3,4). Realize the same by assuming a 	3*4*2 PLA is available.														(10)

b.	Write the behavioural model description of four-to-one multiplexer.				(5)