Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	CONTROL SYSTEMS									       Time: 3 hours

Subject Code:	EE259		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define a linear system.

2.	State Mason’s gain formula.

3.	Define Rise Time.

4.	How is the system classified depending on the value of damping?

5.	What is cut of rate?

6.	Define corner frequency.

7.	What is root locus?

8.	What is the necessary condition for stability?

9.	What is state space?

10.	What are phase variables?

PART – B (5 x 3 = 15 MARKS)

11.	Differentiate between open loop and closed loop control system.

12.	The damping ratio of system is 0.6 and the natural frequency of oscillation is 8 rad/sec. Determine the rise time.

13.	What are the advantages of bode plot?

14.	Verify the given system is stable (or) unstable s4+8s3+18s2 +16s+5=0.

15.	Write the canonical form of state model of nth order system.

PART – C (5 x 15 = 75 MARKS)

H2

H1

G2

G3

G1

G4

R(s)

C(s)

+

+

+

+

+

-

-

16.	Find the transfer function of the given block diagram using block diagram reduction technique.

(OR)

R(s)

C(s)

G1

G2

G3

G4

G5

1

-H4

-H3

-H2

-H1

H5

H6

17.	Find the overall gain of the system whose signal flow graph is shown in figure.

[P.T.O]

18.	Obtain the time response of second order under damped system when it is subjected to unit step input.

(OR)

19.	The open loop transfer function   of a system with unity feedback is G(s) = 10/s (0.1s+1).  Find the static error constants of the system. Obtain the steady state error of the system, when subjected to an input given by the polynomial r(t)=a0+a1t+a2 t2/2.

20.	Sketch the Bode plot for the following transfer function and determine phase margin and gain margin.

G(s) = 10/s (1+0.4s) (1+0.1s)

(OR)

21.	Sketch the polar plot and determine phase margin and gain margin G(s) =1/s2(1+s) (1+2s).

22.	Using R-H method, determine the stability of the system.

a.	s6+2s5+8s4+12s3+20s2+16s+16 = 0		b.	s7+9s6+24s5+24s3+24s2+23s+15 = 0

(OR)

23.	A unity feedback system has open loop transfer function G(s) =K/s (s2+4s+13). Sketch the root locus and analyze stability of the system.

24.	Obtain the state model of the system whose transfer function is given by

Y(s)/ U(s) = 10/(s3 +4s2+2s+1) using phase variable form.

(OR)

25.	Obtain the state model of the system whose transfer function is given by

Y(s)/ U(s)  =  10 (s+4)/ s (s+1) (s+3). Use canonical variables.