Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MEASUREMENTS AND INSTRUMENTATION			       Time: 3 hours

Subject Code:	EE262		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The unit of charge is _______.

2.	The equation for the developed torque, derived from the basic law for electromagnetic torque is _______.

3.	The unit of _______ is watt.

4.	What are the errors in Dynamometer type wattmeter?

5.	Maxwell’s Bridge is suited for accurate measurement of _______.

6.	Anderson Bridge is used for precise measurement of _______.

7.	What is an active transducer?

8.	Thermistors have a _______ temperature coefficient.

9.	List any two advantages of Digital Voltmeter.

10.	What is distortion factor?

PART – B (5 x 3 = 15 MARKS)

11.	A moving coil instrument has the following data.

Number of turns=100, Width of the coil=20mm, Depth of the coil=30mm, Flux density in the air gap=0.1 wb/m2. Calculate the deflecting torque when carrying a current of 10mA. Also calculate the deflection, if the control spring constant is 2x10-6 Nm/degree.

12.	Write a note on Power factor meter.

13.	The impedances of the basic ac bridge are given as follows:

Z1 = 100 Ω 800 (inductive impedance)

Z2 = 250 Ω (pure resistance)

Z3 = 400 Ω 300 (inductive impedance)

Z4 = unknown

Determine the constants of the unknown arm.

14.	List the factors for the selection of transducer.

15.	Write a note on function generator.

PART – C (5 x 15 = 75 MARKS)

16.	Explain about attraction type moving iron instrument with construction, working and derivation of torque equation.

(OR)

17.	Explain about moving coil instrument.

18.	With neat diagram explain the working of a maximum demand indicator.

(OR)

19.	Explain about induction type energy meter and calibration of energy meter.

[P.T.O]

20.	a.	Explain about Wheatstone bridge.												(7)

b.	Explain about Kelvin’s double bridge.											(8)

(OR)

21.	a.	Explain about Hay’s Bridge.													(8)

b.	Explain the working principle of Schering’s Bridge.								(7)

22.	Write short notes on:	a.	LVDT	b.	Strain Gauge									(7+8)

(OR)

23.	Write short notes on:	a.	Resistance thermometers		b.	Thermocouples			(8+7)

24.	Draw the block diagram of Digital Multimeter and explain.

(OR)

25.	Write short notes on:	a.	Strip Chart Recorder	b.	X-Y Recorder