Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	CONTROL SYSTEMS									       Time: 3 hours

Subject Code:	EI203		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define transfer function.

2.	What is the use of synchro?

3.	Distinguish between type and order of a system.

4.	What is meant by frequency response?

5.	How do you define BIBO stability?

6.	What is the stability condition at which the roots are in imaginary axis in S plane?

7.	What is the use of polar plot?

8.	What is Nichols chart?

9.	Write the equation for PI controller.

10.	Name any two applications of fuzzy controller in computer-based control?

PART – B (5 x 3 = 15 MARKS)

11.	Write Mason’s gain formula.

12.	What are the frequency domain specifications?

13.	State and explain Routh stability criterion.

14.	How phase margin is determined from Bode plot?

15.	What are the advantages of fuzzy controller compared to PID controller?

PART – C (5 x 15 = 75 MARKS)

16.	Find the overall transfer function of the system whose signal flow graph is shown in figure.

G4

G1

G2

G3

X

Y

-H1

-H2

-H3

(OR)

17.	Using block diagram reduction technique find closed loop transfer function of the system whose block diagram is shown in figure.

[P.T.O]

18.	The open loop transfer function of a servo system with unity feedback is G(s) =10/s (0.1s+1). Evaluate the static error constants of the system. Obtain the steady state error of the system when subjected to an input given by the polynomial, r(t)=a0 +a1 t+a2 /2 t2.

(OR)

19.	Derive the expressions for frequency domain specifications of a second order system.

20.	Determine the range of K for stability of unity feedback system whose open loop transfer function is G(s) = K/s(s+1)(s+2).

(OR)

21.	Sketch the root-locus of G(s) = k/(s2+10s+100).

22.	Sketch Bode plot for the following transfer function and determine the system gain K for the gain cross over frequency to be 5 rad/sec.  G(s) = Ks2 / (1+0.2s) (1+0.02s)

(OR)

23.	Draw the Nyquist plot for the system, whose open loop transfer function is,

G(s)H(s)= K/s(s+2) (s+10).  Determine the range of K for which closed loop system is stable.

24.	Explain how fuzzy controller used for temperature control applications.

(OR)

25.	Explain the design procedure for PD controller using time domain specifications.