Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MICROPROCESSORS AND MICROCONTROLLERS	       Time: 3 hours

Subject Code:	EI237		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define T-state.

2.	How is program counter useful in program execution?

3.	List the four categories of 8085 instruction that manipulate data.

4.	What is Key bouncing?

5.	What are handshaking signals?

6.	List the operating modes of 8255 Programmable Peripheral Interface.

7.	List the methods for servicing the interrupts.

8.	What are the needs for interrupts?

9.	What is the function of Watch Dog Timer?

10.	What is the function of DPTR register in 8051?

PART – B (5 x 3 = 15 MARKS)

11.	How does a microprocessor differentiate between data and instruction code?

12.	List the different methods to clear the content of accumulator.

13.	Differentiate between memory mapped I/O scheme and I/O mapped I/O scheme.

14.	Discuss the methods for servicing an interrupt.

15.	Differentiate between microprocessor and microcontroller.

PART – C (5 x 15 = 75 MARKS)

16.	Explain the architecture of 8085 microprocessor with a neat diagram.

(OR)

17.	a.	Calculate the COUNT value to be loaded in order to produce a delay of 250µs		(8)

MVI D, AAH

ROTATE:	MOV A, D

RLC

MOV D, A

ANI 01H

OUT PORT1

MVI B, COUNT

DELAY: 	DCR B

JNZ  DELAY

JMP ROTATE

b.	Draw the timing diagram for MVI A, 46H.										(7)

18.	Explain various addressing modes of 8085 with an example.

(OR)

[P.T.O]

19.	a.	Write an 8085 ALP to add the two eight-bit numbers.

b.	Write an 8085 ALP to separate odd and even numbers. Store the even numbers in 8000H and odd numbers in 9000H.			                                                                 	(5)

20.	With a neat block diagram, explain the architecture of PPI.

(OR)

21.	Explain the interfacing of keyboard with 8085.

22.	With neat diagram, explain the stepper motor control using 8085.

(OR)

23.	Explain the need for Direct Memory Access, devices for handling DMA and typical DMA controller features.

24.	Explain the architecture of 8051 microcontroller with a neat diagram.

(OR)

25.	Explain the various addressing modes of 8051 microcontroller with examples.