Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	SENSORS AND TRANSDUCERS						       Time: 3 hours

Subject Code:	EI247		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Distinguish between the direct and indirect methods of measurement.

2.	Mention the types of errors.

3.	What is function of an inverse transducer?

4.	List any two passive transducers.

5.	Give an application of microsyn and synchro.

6.	Thermistor has _______ temperature coefficient.

7.	_______ and _______ belong to natural group material, which exhibits piezo-electric effect.

8.	Mention any one disadvantage of capacitive transducer.

9.	Digital speed transducer is used for _______.

10.	What is difference between sensor and smart sensor?

PART – B (5 x 3 = 15 MARKS)

11.	Draw and explain the basic blocks of generalized instrumentation systems.

12.	Mention the factors influencing the choice of transducers.

13.	Define gauge factor.

14.	Draw the frequency response of capacitive transducer.

15.	Explain the basic generic styles of optical encoder.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Define the terms (i) repeatability (ii) accuracy	(iii) precision and (iv) resolution.	(8)

b.	A 0 -150 V voltmeter has a guaranteed accuracy of 1 percent of full scale reading. The 	voltage measured by this instrument is 75V. Calculate the limiting error in percent.	(7)

(OR)

17.	a.	Define the term standards and explain the classification of standards.			   	(10)

b.	The dead zone in a certain pyrometer is 0.125 percent of span. The calibration is 400ºC to 	1000ºC. What temperature change might occur before it is detected.			   	(5)

18.	Differentiate 		a.	analog and digital transducer 						   			(8)

b.	primary and secondary transducer					   			(7)

(OR)

19.	a.	Explain the transfer characteristics of transducer.					 	   			(8)

b.	Derive the first order mathematical model of transducer with unit step input.		   	(7)

20.	a.	Mention the types of strain gauges and explain any one of them.				   	(8)

b.	Explain the hot junction compensation circuit.						   			(7)

(OR)

21.	Give the principle and working of LVDT and variable reluctance accelerometer.

[P.T.O]

22.	a.	Explain the working of capacitive transducer using the change in distance between 	plates.																		(9)

b.	Mention the properties of piezo-electric crystals.						    			(6)

(OR)

23.	Explain the modes of operation of piezo-electric crystals.

24.	Write a short note on vibration and sound sensors.

(OR)

25.	Explain in detail (a) fibre optic sensor and (b) pH sensor.