Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DIGITAL CONTROL SYSTEMS						       Time: 3 hours

Subject Code:	EI259		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Sampling frequency must be at least _______ the highest frequency present in the signal.

2.	What is discrete time impulse?

3.	What is delay element?

4.	Give final value theorem.

5.	What is pulse transfer function?

6.	Mention Cayley-Hamilton theorem.

7.	If all poles are lying inside the unit circle the system is said to be _______.

8.	A matrix, which has the corresponding characteristic root and super diagonal has all 1’s is called ________.

9.	A pure derivative should not be implemented because it will give_______.

10.	Give the Dahlin controller equation.

PART – B (5 x 3 = 15 MARKS)

11.	State the two problems associated with the use of sampling theory.

12.	What is aliasing?

13.	What is FIR system?

14.	When do you use quantization?

15.	What is ringing?

PART – C (5 x 15 = 75 MARKS)

16. 	Explain the principles of signal conversion in detail.

(OR)

17.	a.	Give the details of the transformation of discrete time sequences into analog signals. 	(8)

b.	Discuss zero order hold in detail.												(7)

18.	a.	Find the response at the sampling instants for the function  Y(Z)=0.3678z+0.2642z2−1.3678z+0.3678

b.	Consider a discrete time system y(k+2)+14y(k+1)−18y(k)=3r(k+1)−r(k)With input 	r(k)=(-1)k µ(k) and initial condition y(-1)=5,y(-2)=-6. Find the output y(k), k≥0.

(OR)

19.	Determine the transfer function for the following system y(k+2)+a1y(k+1)+a2y(k)=b0r(k+2)+b1r(k+1)+b2r(k), assuming zero initial conditions.

20.	Discuss three types of recursive realisations in implementation of digital controllers.

(OR)

[P.T.O]

21.	A feedback system has a closed loop transfer function Y(s)R(s)=10(s+4)s(s+1)(s+3)

a.	One where the system matrix A is a diagonal matrix

b.	One where A is in first companion form.

c.	One where A is in second companion form.

d.	Construct three different state models for this system.

22.	a.	Explain the Jury stability criterion in detail.

b.	Determine whether the given system is stableΔ(z)=2z4+7z3+10z2+4z+1.

(OR)

23.	Explain the technique, which is used to derive a discrete system from an analog system.

24.	Explain deadbeat controller in detail.

(OR)

25.	Explain any practical system uses digital controller for its operation.