Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title : 	DIGITAL SIGNAL PROCESSING						      Time : 3 hours

Subject Code:	EI262			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Give some applications of DSP.

2.	Compare s-domain and z-domain

3.	What are FIR filters?

4.	Define Gibb’s phenomenon.

5.	How is the analog frequency mapped to digital frequency in bilinear transformation?

6.	Compare von Neumann architecture and Harvard architecture.

7.	What is the function of ARAU?

8.	Is TMS 320c5x is fixed point or floating point processor.

9.	Name some special addressing modes in DSP processors.

10.	What do you mean by RISC?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the similarities and differences between DIT and DIF algorithms.

12. 	Write the steps involved in FIR fitter design.

13.	Find H (z) by using bilinear transformation to H(s)=2(s+1)(s+2)with T = 1 sec.

14.	Draw the block diagram of multiplier accumulator unit.

15. 	How is the repeat count determined in the RPT and RPT B instructions of C5x?

PART – C  (5 x 15 = 75 MARKS)

16. 	Obtain the DFTs using 8 point DIT algorithm of the sequence x(n) = {1, 1, 1, 1, 1, 1, 1, 1}

(OR)

17. 	a.	Perform periodic convolution of two sequences x (n) = {1, 2, 1, 2} and h(n) = {2, 4, 2, 1}

b.	Mention the advantages of FFT over DFT.

18.	Design a low pass fitter using rectangular window by taking 9 samples of w(n) and with a cutoff frequency of 1.2 radians / sec.  Assume suitable sampling frequency.

(OR)

19. 	Design a highpass fitter using hamming window, with a cut-off frequency of 1.2 radians/sec and N=9.  Assume suitable sampling frequency.

20. 	Design a digital Butter worth fitter satisfying the following constraints using bilinear transform. Assume T = 1 sec.

12≤|H(w)|≤1.0; 0≤w≤0.2π |H(w)≤0.08;0.4π≤w≤π

(OR)

[P.T.O]

21. 	Obtain the cascade and parallel form realizations of the LTI system governed by the equation

y(n)=−38y(n−1)+332y(n−2)+164y(n−3)+x(n)+3x(n−1)+2x(n−2).

22. 	 a.	What is meant by bit reversed addressing and circular addressing mode? What is the

application for which this addressing mode is preferred?

b.	List the relative merits and demerits of RISC and CISC Processors.

(OR)

23. 	Explain what is meant by instruction pipelining. Explain with an example how pipelining increases the through put efficiency.

24. 	 Write an ALP in C5x to convolve two sequences of length 4 & 3 respectively using MAC instruction.

(OR)

25. 	 With a neat diagram, explain the architecture of TMS320c5x processor.