Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	COMMUNICATION ENGINEERING					       Time: 3 hours

Subject Code:	IT241		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	A signal is amplified 100 times in power. Calculate the gain in dB.

2.	Define Noise figure.

3.	A signal is band limited to 12 kHz. What is the Nyquist sampling rate required?

4.	What is the function of an Antenna?

5.	How many bits are used to represent a character in ASCII?

6.	What do line drivers and receivers do?

7.	What does DSL stand for?

8.	What does a modem do?

9.	What is the function of a communication satellite?

10.	What does NAVSTAR stand for?

PART – B (5 x 3 = 15 MARKS)

11.	What is modulation? Give two reasons why modulation is needed.

12.	Compare FM with PM.

13.	Compare bit rate with baud.

14.	What is interlacing? Why is it done?

15.	What is cell splitting? When is it done and why?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Distinguish between low level modulation and high level modulation.   				(5)

b.	Draw the block diagram of low level modulation transmitter and high level modulation 	transmitter and explain their working.                                 							(10)

(OR)

17.	Draw the block diagram of superheterodyne radio receiver and explain its operation. What are its advantages?

18.	Explain any one method of generation of FM wave and demodulation of FM wave.

(OR)

19.	Write notes on the following:

a.	Log-periodic antenna                   b.	Dish antenna							               (8+7)

20.	Describe the following error detection techniques with suitable examples:

a.	Parity bit		b.	Checksum												(6+9)

(OR)

21.	What is delta modulation? Draw the block diagram of a delta modulation system and explain its 	operation. Also sketch its waveforms.

22.	What are the four functional groupings of RS-232 signal lines? What are their roles?

(OR)

[P.T.O]

23.	Draw the block diagram of a color TV receiver and explain its working.

24.	List the various segments of a GPS system. Explain their functions in detail.

(OR)

25.	List the various subsystems of satellite earth station. Explain their functions.