Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	ENGINEERING THERMODYNAMICS					       Time: 3 hours

Subject Code:	ME228		       						               	       Maximum Marks: 100

#### Answer ALL questions

(Use of approved Thermodynamics property tables allowed)

PART – A (10 x 1 = 10 MARKS)

1. 	Which law is used as the basis for temperature measurement?

2.	How do intensive properties differ from extensive properties?

3.	Entropy of universe always decreases due to energy utilization.  (True / False)

4.	Relate the COP of heat pump to COP of a refrigerator.

5.	State the phase rule for a system at equilibrium.

6.	Define wetness fraction of steam.

7.	Differentiate ideal and real gases.

8.	Write down the formula for RMS velocity of gas molecules.

9.	Draw the T-s diagram for dual cycle.

10.	What is the optimum intercooler pressure in the gas turbines?

PART – B (5 x 3 = 15 MARKS)

11.	Compare open, closed and isolated thermodynamic systems.

12.	State Kelvin-Planck and Classius statements of second law of thermodynamics.

13.	Draw the T-v and p-T diagrams of a pure substance.

14.	Find higher calorific value, lower calorific value and minimum air required for complete combustion of 1 kg of a low-grade fuel with C=42%, H2=8% and remaining incombustible materials.

15.	In a Diesel cycle, the compression ratio is 13 and cut-off volume is 1.96 m3 while the stroke volume is 12 m3. Find the thermal efficiency if specific heat ratio for air as 1.4.

PART – C (5 x 15 = 75 MARKS)

16.	The values of specific heats at constant pressure and at constant volume for an ideal gas are 0.984 kJ/kgK and 0.728 kJ/kgK respectively. Find the characteristic gas constant and ratio of specific heats for the gas. If one kg of this gas is heated at constant pressure from 25C to 200C, estimate the heat added, ideal work done and change in internal energy. Also calculate the pressure and final volume if the initial volume was 2 m3.

(OR)

17.	One kg of fluid enters a nozzle with a velocity of 3300 m/min and enthalpy of 2990 kJ/kg. The enthalpy of the fluid at exit is 2760 kJ/kg. The nozzle is placed horizontally; neglect the heat loss from the nozzle. Determine a. velocity of fluid at exit, b. mass flow rate if the inlet area of nozzle is 0.095 m2 and specific volume at inlet is 0.19 m3/kg,      c. exit area of  nozzle if the specific volume at exit is 0.5 m3/kg.

18.	A reversible engine is supplied with heat from two constant temperature sources at 900 K and 600 K and rejects heat to a constant temperature sink at 300 K. The engine develops work equivalent to 90 kJ/s and rejects heat at the rate of 56 kJ/s. Estimate a. Heat supplied by each source 	b. Thermal efficiency of the engine.

[P.T.O]

(OR)

19.	Calculate the decrease in availability when 20 kg of water at 100 C mix with 30 kg of water at 40 C, the pressure being taken as constant and the temperature of the surrounding being 20 C. Take Cp of water = 4.187 kJ/kg K.

20.	A sample of steam at 200 C and a pressure of 7 bar is first expanded at a constant enthalpy to 3 bar, and then at constant entropy to 0.5 bar. Using Mollier chart, determine the change in entropy and enthalpy during the processes.

(OR)

21.	Describe the following with suitable diagram:

a.	T-s diagram of steam formation		b.	h-s diagram of steam formation

22.	A mixture contains 2 kg of N2 and 4 kg of CO2 at 300K and 1 bar. Take gas constant for         N2 = 0.26 kJ/kg-K and CO2 = 0.20 kJ/kg-K. Find the mole fraction, partial pressures, partial volumes and equivalent gas constant of the mixture.

(OR)

23.	Explain the working of Bomb’s calorimeter with a neat sketch.

24.	Pressure and temperature of air at the entry to an Otto cycle are 1 bar and 300 K respectively. The compression ratio is 7.5. If the temperature of air after expansion is 550 K, calculate the thermal efficiency, the maximum temperature and maximum pressure and the mean effective pressure of the cycle.

(OR)

25.	Using Mollier chart, find the enthalpy drop and final condition of steam when it is expanded isentropically from an initial pressure of 30 bar and 350 C to a pressure of 1 bar. Also find the work output of the turbine if mass flow rate of steam is 10 kg/s if turbine efficiency is 88%.