Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	THERMAL ENGINEERING							       Time: 3 hours

Subject Code:	ME229		       						               	       Maximum Marks: 100

#### Answer ALL questions

(Approved Steam Tables, Mollier Diagrams, Psychrometric Charts are permitted)

PART – A (10 x 1 = 10 MARKS)

1. 	What do you mean by scavenging in IC engines?

2.	State any four functions of lubrication.

3.	Define nozzle efficiency.

4.	What is the need of compounding in steam turbines?

5.	What is meant by free air delivered?

6.	Give any two merits of rotary compressor over reciprocating compressor.

7.	What are the properties of good refrigerant?

8.	Define tons of refrigeration.

9.	Define relative humidity.

10.	What are the requirements of comfort air conditioning system?

PART – B (5 x 3 = 15 MARKS)

11.	Compare two-stroke and four-stroke engine.

12.	What are the effects of friction on the flow through a steam nozzle?

13.	What are the effects of clearance volume?

14.	Draw the PV and TS diagram for vapour compression refrigeration cycle for super heating process.

15.	On a psychrometric chart, show all the property lines.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Describe the port timing diagram of two stroke IC engine with neat sketch. 			(8)

b.	Describe the working principle of EGR system with a neat sketch. 		       			(7)

(OR)

17.	Mention the various methods of lubrication system and explain any two in detail.

18.	a.	Steam approaches a nozzle with a velocity of 200 m/s, pressure of 4 bar and dryness 	fraction of 0.98. If the isentropic expansion in the nozzle proceeds till the pressure of exit 	is 1 bar. Determine the change in enthalpy and dryness fraction of steam using mollier 	diagram. Calculate also the exit velocity of steam from nozzle and exit area of nozzle for 	flow of 0.8 kg/s.

(OR)

19.	Describe the working principle of impulse and reaction turbine with a neat sketch.

20.	Explain the principle of operation single reciprocating compressor with a neat sketch.

(OR)

[P.T.O]

21.	A single stage, single acting air compressor has a cylinder bore of 175 mm and a stroke of 225 mm. The compressor sucks air at 100 kN/m2 and 27oC and delivers at 8 bar. Find the power required to drive a compressor if its speed is 120 rpm. Also calculate the mass of air compressed per minute and delivery temperature. The compression follows the law PV1.3 = C.

22.	With a neat sketch, explain the working principle of lithium bromide refrigeration system in detail.

(OR)

23.	Explain the working principle of ammonia water refrigeration system in detail with neat sketch.

24.	An air conditioning system is designed under the following conditions

Outdoor conditions – 30oC DBT, 75% RH

Required indoor conditions – 22oC DBT, 70 % RH

Amount of free air circulated – 3.33 m3/s

Coil dew point temperature – 14oC

The required indoor condition is achieved first by cooling and dehumidification, and then by heating. Estimate (a) the capacity of cooling coil in tonnes and (b) the capacity of heating coil in kw.

(OR)

25.	Air at 20oC, 40% relative humidity is mixed adiabatically with air air at 40oC, 40% relative humidity in the ratio of 1 kg of the former with 2 kg of the latter (on dry basis). Find the final condition of air.