Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	HEAT AND MASS TRANSFER							       Time: 3 hours

Subject Code:	ME230		       						               	       Maximum Marks: 100

#### Answer ALL questions

(Use of approved HMT tables permitted)

PART – A (10 x 1 = 10 MARKS)

1.	What is heat flux?

2.	What is the unit for overall heat transfer coefficient?

3.	Define Biot number.

4.	What is conduction shape factor?

5.	What is the Reynolds number for turbulent flow?

6.	In flow over a flat plate the convection coefficient decreases, increases and decreases.

(True / False)

7.	In the counter flow heat exchangers the hot and cold fluids flow in the same direction. (True / False)

8.	What the different modes of mass transfer?

9.	What are the various types of condensation?

10.	Opaque Gray surface have constant reflectivity. (True / False)

PART – B (5 x 3 = 15 MARKS)

11.	What are the assumptions made for Fourier’s law of heat conduction?

12.	What is a lumped heat capacity system?

13.	What are the conditions which help the flow to be laminar?

14.	What is fouling? How is it taken care of?

15.	What is boiling heat transfer?

PART – C (5 x 15 = 75 MARKS)

16.	Derive an expression for temperature distribution under one dimensional steady state heat Conduction and without heat generation for a hollow cylinder.

(OR)

17.	The wall of a building in a composite consisting of 250 mm layer of common Brick (k=0.72W/mK) and 10mm layer of Gypsum plaster (k=0.12W/mK) on both the Side of the bricks. During a hot day (at steady state), the temperature of the outside Plaster (exposed to ambient air) is 40C and the temperature of the inside plaster (exposed to inside air) is 25C. Find (a) the heat flow rate through the wall per unit area of the wall. (b) the temperature of the interface of the brick and outside plaster.

18.	Find the expression of cooling of a lumped body.

(OR)

19.	An egg with mean diameter of 4cm is initially at 25C. It is placed in boiling water For 4 minutes and found to be consumer’s taste. For how long should a similar egg for same customer be boiled when taken from refrigerator at 2C. Use lumped system  Analysis and take thermo physical properties of egg as; k=12W/m2K,Cp=2000J/kgK Density=1250kg/m3.

[P.T.O]

20.	Explain the concept of velocity boundary layer development over a flat plate.

(OR)

21.	Water at 10C is to be heated to 40C in a tube of 2cm diameter. The outside of the tube is wrapped with an electric heating element that produces a uniform flux of 200N/m3 over the surface and the mass flow rate is 0.2kg/s. Find										(3x5)

a.	The Reynolds number		b.	The heat transfer coefficient

c.	Length of pipe

The relevant properties of water at 25C are given below

Density=997kg/m3 	Cp=4180J/kgK

K=0.608W/mK		µ=0.00091poise

22.	a.	Briefly discuss about the types of heat exchangers.								(10)

b.	Derive an expression for LMTD for counter-flow heat exchangers.					(5)

(OR)

23.	a.	Explain Fick’s law of diffusion.												(10)

b.	Differentiate between heat transfer and mass transfer.								(5)

24.	Explain the various regions of the saturated pool boiling.

(OR)

25.	a.	Explain absorptivity, reflectivity and transmissivity.								(10)

b.	Explain about greenhouse effect.												(5)