Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DESIGN OF MECHANICAL TRANSMISSION SYSTEM	   Time: 3 hours

Subject Code:	ME231		       						               	       Maximum Marks: 100

#### Answer ALL questions

(Use of approved data books permitted)

PART – A (10 x 1 = 10 MARKS)

1. 	What is thrust bearing?

2.	What is optimum belt velocity for maximum power transmission?

3.	Define pitch of a chain.

4.	What is herringbone gear?

5.	What is crown gear?

6.	How do you specify worm and worm wheel?

7.	Give structural formula for gear box.

8.	Give the structural formula for 14 speed gear box.

9.	Define pressure angle in cam.

10.	State the applications of Ratchet and Pawl mechanism.

PART – B (5 x 3 = 15 MARKS)

11.	List out the applications of sliding and rolling contact bearings.

12.	What are the advantages of wire rope?

13.	What are the forces acting on bevel gear?

14.	Give the classifications of speed reducers.

15.	How is undercutting occurs in cam?

PART – C (5 x 15 = 75 MARKS)

16.	Design a suitable journal bearing for a centrifugal pump from the following available data: Load on the bearing 13.25 kN. Diameter of the journal = 80 mm, RPM = 1440.Bearing characteristic number = 30 X 10-6. Permissible bearing pressure = 0.7 to 1.4 N/mm2.Average atmospheric temperature = 30˚C.Calculate the cooling requirements using Lasche’s equation. Use Mckee’s equation for calculating the friction coefficient.

(OR)

17.	Design a V belt drive and calculate the actual belt tensions and average stress for the following data. D = 500 mm, d= 150 mm, C = 925 mm, n1= 1000 rpm, n2 = 300 rpm and P = 7.5 kW.

18.	Design a chain drive to transmit 6 kW at 900 rpm of a sprocket pinion. Speed reduction is 2.5:1.driving motor is mounted on an adjustable base. Assume that load is steady, drive is horizontal and service is 16 hours/day.

(OR)

19.	A compressor running at 360 rpm is driven by a 150 kW, 1440 rpm motor through a pair of 20˚ FD helical gears having helix angle of 25˚. The centre distance is approximately 400 mm. the motor pinion is to be forged steel and the driven gear is to be cast steel. Assume medium shock conditions. Design the gear pair.

[P.T.O]

20.	Design a pair of CI bevel gears for a special purpose machine tool to transmit 3.5 kW from a shaft at 500 rpm to another at 800 rpm. The gears overhang in their shafts. Life required is 8000 hours.

(OR)

21.	Design a worm gear drive with a standard centre distance to transmit 7.5 kW from a worm rotating at 1440 rpm to a worm wheel at 20 rpm.

22.	Draw the kinematic diagram and speed diagram of the head stock gear box of a turret lathe having arrangement for 9 spindle speeds ranging from 31.5 rpm to 1050 rpm. Calculate the number of teeth on each gear. Minimum number of teeth on a gear is 25. Also calculate the percentage deviation of the obtainable speeds from the calculated ones.

(OR)

23.	A gearbox is to designed with the following specifications: Power = 14.72 kW. Number of speeds = 18. Minimum speed 16 rpm. Speed ratio = 1.25. Motor speed = 1400 rpm. The 18 speeds are obtained as 2 X 3 X 3. Sketch the layout of the gearbox and the speed diagram. Calculate the diameter of the shafts and the number of teeth on the gears.

24.	Determine the geometric dimensions of Geneva wheel for the driving crank radius of 50 mm. The Geneva wheel has 6 slots.

(OR)

25.	A differential screw jack is shown in figure. Neither screw rotates. Outside diameter of the screw is 50 mm. Single start square thread is used. Coefficient of friction is 0.15. What is the efficiency of the jack? What load can be raised if the shear stress in the screw is limited to 25 N/mm2.Assume torque applied to the nut does not cause bending stress in the lower screw.