Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title : 	MECHANICS OF MACHINES - I						      Time : 3 hours

Subject Code:	ME254		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define kinematic pair.

2.	List out the various inversions of double slider crank chain.

3.	What is the functional difference between clutch and bake?

4.	Define gyroscopic effect.

5.	Name the essential members of a cam mechanism.

6.	What are the different motions of follower in cam mechanism?

7.	Define module and pressure angle of gears.

8.	What is reverted gear train?

9.	What is the main function of governor?

10.	Mention any three types of spring loaded governor.

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain various types of constrained motions.

12.	What is self-locking and self-energized bake?

13.	State the basic requirements for high speed cams.

14.	Define interference in gears.

15.	Define stability and sensitivity of governor.

PART – C  (5 x 15 = 75 MARKS)

16.	What is meant by inversion of a chain? Sketch and describe the mechanisms obtained by the inversion of four bar chain.

(OR)

17.	The crank AB of slider crank mechanism is 15cm and the connecting rod BC is 60cm long. The crank makes 300 rpm in the clockwise direction. When it has tuned 45º from the inner dead center position, determine :

a.	Linear velocity of slider C

b.	Acceleration of the mid-point of the connecting rod and

c.	Angular acceleration of the connecting rod.

18.	A multi plate clutch transmits 25kW of power at 1600 rpm it has three discs on the driving shaft and two on the driven shaft. Coefficient of friction for the friction surfaces is 0.25. The external and internal radii of friction surfaces are 100 mm and 50mm respectively. Find the maximum intensity of pressure between the discs. Assume uniform wear.

(OR)

[P.T.O]

19.	a.	Explain the gyroscopic effect on an aeroplane.									(10)

b.	The turbine rotor of a ship has a mass of 3500 kg. It has a radius of gyration of 0.45m and a speed of 3000 rpm clockwise when looking from stern. Determine the gyroscopic couple and its effect upon the ship, when the ship is steering to the left on a curve of 100 m radius at a speed of 36 km/h.															(5)

20.	A cam is to give the following motion to a knife-edge follower:

a.	Outstroke during 60° of cam rotation;

b.	Dwell for the next 30° of cam rotation;

c.	Return stroke during next 60° of cam rotation and

d.	Dwell for the remaining 210° of cam rotation.

The stroke of the follower is 40 mm and the minimum radius of the cam is 50 mm. The follower moves with uniform velocity during both the outstroke and return strokes. Draw the profile of the cam when the axis of the follower is offset by 20 mm from the axis of the cam shaft

(OR)

21.	Construct a tangent cam and mention the important terminologies on it. Also derive the expressions for displacement, velocity, acceleration of a reciprocating roller follower when the roller has contact with the nose.

22.	a.	What are the advantages of involute teeth over cycloidal teeth?						(6)

b.	Derive an expression for the length of the arc of contact in a pair of meshed spur gears.

(9)

(OR)

23.	An epicyclic gear train consists of three gears A, B, and C as shown in fig.1. The gear A has 72 internal teeth and gear C has 32 external teeth. The gear B meshes with both A and C and is carried on an arm EF which rotates about the centre of A at 18 rpm. If the gear A is fixed, determine the speed of gears B and C.

24.	A Porter governor has all four arms 250 mm long. The upper arms are attached on the axis of rotation and the lower arms are attached to the sleeve at a distance of 30 mm from the axis. The mass of each ball is 5 kg and the sleeve has a mass of 50 kg. The extreme radii of rotation are 150 mm and 200 mm. Determine the range of speed of the governor.

(OR)

25.	Describe the function of Hartnell governor with neat sketch and deduce a relation to find the stiffness of the spring.