Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MECHANICS OF MACHINES - II 						       Time: 3 hours

Subject Code:	ME255		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the difference between Equilibrium and Equilibrant?

2.	What are the laws of static Friction?

3.	Define coefficient of steadiness.

4.	Draw the turning moment diagram of multicylinder engines.

5.	What is meant by Logarithmic Decrement?

6.	Define critical speed.

7.	What are the conditions for static balancing?

8.	Define hammer blow.

9.	Define Equivalent length of a shaft.

10.	What is a node and antinode?

PART – B (5 x 3 = 15 MARKS)

11.	Define dynamically equivalent system.

12. 	What is flywheel?  Give the functions of a flywheel.

13.	Write short note on partial balancing of reciprocating masses and its effects.

14.	Briefly explain the different types of damping.

15. 	Explain the significance of Holzer’s method.

PART – C (5 x 15 = 75 MARKS)

D

300N

300

C

800

B

A

0

200

T2



400

400

1

2

3

4

1

5

AB = 200 mm

BC = 800 mm

CD = 300 mm

“All dimensions in mm”

16. 	The static equilibrium of the quick return mechanism is shown in figure find the maximum input torque T2 required for a force of 300N on the slider ‘D’.  Angle ‘A’ is 105.  Coefficient of friction 𝜇 = 0.15 for each sliding pair.

 #

(OR)

17. 	The piston diameter of a internal combustion engine is 125 mm and the stroke is 220mm.  Connecting rod is 4.5 times crank length and has a mass of 50kg.  The mass of the reciprocating parts is 30kg.  The centre of mass of connecting rod is 170mm from crank pin centre and radius of gyration about an axis through the centre of mass is 148mm.  the engine inertia force and the corresponding torque on the crankshaft when the angle turned by crank is 140 from inner dead centre.  Use analytical (or) graphical method.

[P.T.O]

18.	An otto cycle engine develops 50kW at 150rpm with 75 explosions per minute.  The change of speed from commencement to the end of the power stroke must not exceed 0.5% mean on either side.  Find the mean diameter of flywheel and a suitable rim cross section having width four times depth so that the hoop stress doesn’t exceed 4MPa.  Assume that the flywheel stores 16/15 times energy stored by rim and work done during power stoke is 1.40times the work done during the cycle.  Density of the rim material is 7200 kg/m3.

(OR)

19. 	The turning moment diagram of a four stroke SI engine may be assumed as four triangles for simplicity, the areas of portions from the line for zero pressure are as follows: Suction stroke = 0.45 x 10-3m2; Compression stroke = 1.7 x 10-3m2; Expansion stroke = 6.8 x 10-3m2; Exhaust stroke = 0.65 x 10-3m2.  Assume a scale 1m2 = 3MNM of energy.  Assuming resisting torque as uniform, find the mass of rim of a flywheel required to keep speed between 202 rpm and 198rpm.  The mean radius of rim is 1.2m.

20. 	A five cylinder inline engine running at 750rpm has successive cranks 144 apart the distance between the cylinders centre lines being 375mm.  The piston stroke is 225mm and the ratio of connecting rod to the crank is 4.  Examine the engine for balance of primary and secondary forces and couples.  Find the maximum values of these and the position of the central crank at which these maximum value occurs.  The reciprocating mass for each cylinder is 15kg.

(OR)

21. 	The three cranks of a three cylinder locomotive engine are all on the same axle and are set at 120.  The pitch of the cylinder is 1m and the stroke of each piston is 0.6m.  The reciprocating masses are 300kg for inside cylinder and 260kg for each outside cylinder and the planes of rotation of the balancing masses are 0.8m from inside crank.  If 40% of the reciprocating parts are to balanced, find:

a.	Magnitude and position of balancing mass required at a radius of 0.6m and

b.	hammer blow per wheel when the axle makes 6rps.

22. 	 A machine part weighing 39N vibrates in viscous medium.  Determine the damping coefficient when a harmonic exciting force of amplitude 49N results in a resonance amplitude of 1.25cm with a period of 0.2 seconds.  If the system is exited by a harmonic force of frequency 4cps, what will be the percentage increase in the amplitude of vibration when damper is removed?

(OR)

23. 	A vertical steel shaft of 15mm diameter is held in tong bearings 1m apart and carries a mass of 15kg @ center.  Eccentricity of CG of disc from centre of rotor is 0.3mm.  Modulus of Elasticity is 200GN/m2, permissible shear stress is 70MN/m2.  Determine

a.	critical speed of shaft		b.	Range of speed over which it is unsafe to run the shaft neglect the mass of shaft.

24. 	 A motor drives a centrifugal pump through gearing, the pump speed being one-third that of the motor.  The shaft from the motor to the pinion is 60mm diameter of 500 mm long.  The moment of inertia of motor is 400 kg-m2.  The impeller shaft is 100mm diameter and 600mm long.  The moment of inertia of impeller is 1500 kg-m2. Neglecting inertia of gears and shaft, find natural frequency of Torsional vibration. (Take C = 800GN/m2)

(OR)

K

2K

3K

4K

4

3

2

1

I1 =4Kgm2

I2 =3Kgm2

I3 =2Kgm2

I4 =1Kgm2

25. 	 Find the Natural frequencies of the four degree of freedom system given in figure using Holzer method.