Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	INSTRUMENTATION AND CONTROL SYSTEMS		       Time: 3 hours

Subject Code:	ME256		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is Calibration?

2.	The Sensing unit is a generalized measurement system for _______ stage.

3.	Thermocouple is a _______ generated device.

4.	Name the transducer used to measure negative pressure.

5.	What are the instruments used to measure torque?

6.	Absorbtion hydrometer is used to measure _______.

7.	_______ are used to add two or more signals in the system.

8.	Which is used to represent the control system graphically?

9.	Write the equation of gain margin.

10.	The _______ is a frequency response plot of the transfer function of a system.

PART – B (5 x 3 = 15 MARKS)

11.	Write short notes on three stages of a generalized measurement system.

12.	Illustrate neatly about bellows pressure gauge.

13.	What are the types of strain measuring techniques?

14.	List the demerits of open loop system.

15.	What is transient and steady state response?

PART – C (5 x 15 = 75 MARKS)

16.	Discuss about recording of measurement data.

(OR)

17.	Write short notes on:  a.	Accuracy		b.	Precision and 	c.	Dynamic terms	(3x5)

18.	Explain the working of Accelerometer with a neat sketch.

(OR)

19.	Discuss the working of ultrasonic special flow meters with neat diagram.

20.	Explain the method used to measure humidity.

(OR)

21.	Explain with neat diagram the working of Piezo-electric transducer.

22.	Draw a signal flow graph and evaluate the closed loop transfer function of a system whose block diagram is shown as

G1

G3

G4

H2

H1

+

-

-

+

-

+

C(s)

R(s)

G2

(OR)

[P.T.O]

23.	What are the advantages and disadvantages of hydraulic devices in control system? Explain hydraulic devices with an example.

24.	Draw the bode plot for the function of GH(s) = 5(1+2s)/(1+4s)(1+0.25s).

(OR)

25.	Use the routh stability criterion to determine the location of roots on the s-plane and hence the stability for the system represented by the characteristic equation s6+s5+3s4+3s3+3s2+2s+1=0.