Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	DESIGN OF MACHINE ELEMENTS					       Time: 3 hours

Subject Code:	ME257		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define Factor of safety.

2.	Define Design.

3.	What is an S-N Curve?

4.	What is the use of Goodman & Soderberg diagrams?

5.	Write the types of shafts.

6.	What types of stresses developed in the key?

7.	What stresses act on screw fastenings?

8.	What is the meaning of bolt M24×2?

9.	What type of external forces act on connecting rod?

10.	Classify crankshaft.

PART – B (5 x 3 = 15 MARKS)

11.	What is stress concentration factor?

12.	Draw the Goodman & Soderberg diagrams and locate the safe design region.

13.	Under what circumstances flexible couplings are used.

14.	Why are welded joints preferred over riveted joints?

15.	What are the stresses set up in an IC engine connecting rod?

PART – C (5 x 15 = 75 MARKS)

16.	Taking stress concentration in to account find the maximum stress induced when a tensile load of 20kN is applied to a stepped shaft of diameters 60mm and 30mm width a fillet radius of 6mm as shown in figure.

(OR)

17.	A steel member is subjected to a 3-D stress system and resulting principal stress are 120N/mm2 tension, 80N/mm2 and 40N/mm2 compression. If the proportional limit of the material in simple tension is 280N/mm2 and its poisson's ratio is 0.3.  Determine the factor of safety according to (a) Maximum principal stress theory (b) Maximum principal strain theory (c) Maximum shear stress theory.

18.	A helical valve spring is to be designed for an operating load range of 90N to 135N.  The deflection of the spring for this load range is 7.5mm.  Assuming a spring index of 10, a permissible shear stress of 480N/ mm2 and a modulus of rigidity of 0.8X l05 N/mm2 for the material, determine the dimensions of the spring.

(OR)

[P.T.O]

19.	The bending stress in a machine part fluctuates between a tensile stress of 280N/mm2 and a compressive stress of 140N/mm2.  What should be the minimum ultimate tensile strength of this part to carry this fluctuation indefinitely according to (a) Goodman's formula (b) Soderberg formula Factor of safety is 1. 75. Assume that the yield point is never likely to be less than 55% of the Ultimate tensile strength or greater than 93 % of it.

20.	A shaft is to transmit 50kW at 1200rpm. It is also subjected to a bending moment of 275 N-m. Allowable shear stress is 60 N/mm2. The shaft is not to twist more than 2° in a length of 2m. Design the shaft. Take G=80×103N/mm2.

(OR)

21.	Design a taper key for a shaft of diameter 100mm transmitting 60kW at 300rpm. The allowable compressive stress may be taken as 175N/mm2.

22.	A double riveted lap joint with zigzag riveting is to be designed for 16mm plates. Let the permissible stresses are 80N/mm2 in tension, 60N/mm2 in shear and 120N/mm2 in crushing, state how the joint will fail and find the efficiency of the joint.

(OR)

23.	A knuckle joint is to transmit a force of 140kN. Allowable stresses in tension, shear and compression are 75N/sq.mm,65N/mm and 140N/sq.mm respectively. Design the joint.

24.	The cylinder of low speed engine is 200mm in diameter and the steam pressure is 0.875MPa. The connecting rod is 1m long and is of rectangular cross section. Width is 2 times the thickness. Find the dimensions of the cross-section.

(OR)

25.	A four –stroke single cylinder gas engine runs at a constant load and delivers 25kW at 300rpm. The maximum fluctuation of energy per cycle may be taken as 0.65 times the useful work per cycle. Design a suitable rim flywheel of rectangular section to limit the variation of speed during the cycle to ± 2% of the mean speed. The flywheel is made of cast iron.