Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MECHANICAL SYSTEMS FOR FOOD PROCESSING	       Time: 3 hours

Subject Code:	ME262		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The centrifugal pump works on the principle of ________.

2.	The major advantage of Peristaltic pump is ________.

3.	Module of a gear tooth is given by ________.

4.	The expression for tension ratio in belt drives is ________.

5.	Economiser is used to recover ________ in boiler.

6.	________ is a door through which the fuel is fed.

7.	The function of expansion valve is to ________.

8.	________ or________ is used as an insulator in ice plants.

9.	Flow rate in material handling refers to ________.

10.	Mention one mechanical device for raising and lowering loads.

PART – B (5 x 3 = 15 MARKS)

11.	Mention the energy losses in centrifugal pumps.

12.	What are the advantages and disadvantages of gear drives over belt drives?

13.	List any six clauses of IBR.

14.	List the different types of water coolers.

15.	Distinguish between product and process layouts.

PART – C (5 x 15 = 75 MARKS)

16. 	A centrifugal pump delivers water against a net head of 16 m and a design speed of 800 rpm. The vanes are curved back to an angle of 300 with the periphery. The impeller diameter is 250 mm and outlet width 40 mm. Determine the discharge of the pump if manometric efficiency is 94%.

(OR)

17.	Explain the working of a screw pump and an axial piston pump with the aid of neat sketches.

18.	Determine the power transmitted and torque for an open belt drive connecting two pulleys 1100 mm and 450 mm diameters, on parallel shafts 3.5m apart. The maximum tension in the belt is 1600 N. The coefficient of friction is 0.3. The driver pulley of diameter 1100 mm runs at 200 rpm.

(OR)

19.	a.	Explain the features and applications of any six types of gear. 						(8)

b.	Describe the modes of gear failure. 												(7)

20.	Sketch and describe the constructional features and working of a Babcock and Wilcox boiler.

(OR)

21.	Explain the working of fusible plug and spring loaded safety valve with sketches.

22.	With the aid of a schematic diagram, explain the working of an ice plant.

[P.T.O]

23.	Explain, with sketches, the pressure type water cooler and remote type cooler.

24.	List and explain the principles of material handling.

(OR)

25.	Explain the working principles of different types of elevators.