Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	PRINCIPLES OF MECHANICAL VIBRATIONS 		       Time: 3 hours

Subject Code:	ME350			         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	An electronic instrument of mass 20kg is subjected to a shock in the form of a step velocity 	of     2m/s.  If the maximum allowable values of deflection (due to clearance limit) and 	acceleration are specified as 20mm and 25g, respectively.  Determine the spring constant 	of an undamped shock isolator.																		(8)

b.	During operation, a 200kg machine is subjected to a 1000N reversed loading.

i)	If machine is mounted in a elastic pad with a stiffness of 300kN/m and a damping 	ratio 	of 0.1, what is the maximum displacement of the machine?						(6)

ii)	It is desired to hold the amplitude of vibration of a machine to 1.5cm and limit the 	transmitted 	force to 5000N.  Design an isolation system with a damping ratio of 0.1 to 	achieve these 	goals?																	(6)

2.	a.	An exhaust fan rotating at 1000rpm is to be supported by four springs each having a 	stiffness of 	‘K’.  If only 10% of unbalanced force of fan is to be transmitted to the base, 	what should be the 	value of ‘K’?  Assume the mass of exhaust fan to be 40kg.								(8)

b.	A machine of a mass 100kg is mochted on springs and damper as shown in figure.  The total spring stiffness is 50,000 N/m and damping factor G = 0.20.  A harmonic force F = 500 sin13.2t acts on the mass. For the sustained (or) steady state vibrations of the system, determine:			(4x3)

i)	Amplitude of the motion of machine		ii)	Its phase with respect to the exciting force

iii)	the transmissibility			iv)	the maximum dynamic force transmitted to the foundation.

M

F

K/2

K/2

(OR)

3.	A 300kg machine is placed at the end of the cantilever beam 1.8m, elastic modulus 200GPa, mement of inertia 1.8 x10-5m4. When the machine operates at 100rpm, it has a steady state amplitude of 0.8mm.  What is the steady state amplitude of the machine when a 30kg absorber of damping coefficient of 650Ns/m and stiffness (1.5x105 N/m) is added at the end of the beam?

x

K1

K2

l1

l2



4.	A machine having a mass of 1500kg and a mass moment of inertia J0 = 400 kgm2, is supported on elastic supports as shown in figure.  If the stiffness of the supports are K1 = 3000 N/mm and K2 = 2000N/mm, and the supports are located from centre of gravity at l1 = 0.4m and l2 = 0.5m, determine the natural frequencies and mode shapes of the machine tool.

(OR)

[P.T.O]

K

K

K

L/2

L/2

L/2

L/4

L/4

X1

X2

5.	Derive the stiffness matrix for the vibrating system shown in figure using stiffness influence coefficients.

6.	A uniform string is tightly stretched between x =0 and x = l is plucked at x = l/4, through a distance ‘h’ and then released from rest find its subsequent displacement.

(OR)

A = 3x10-6m2

E = 200GPa

W = 7800kg/m3

2x105N/m

7.	a.	Determine the lowest natural frequency of longitudinal motion for the system shown in 	figure.																					(7)

2m

M1

M2

M3

X1(t)

X2(t)

X3(t)

b.	Estimate the fundamental frequency of vibration of the system shown in figure.  Assume

M1 = M2 = M3 = M, K1 = K2 = K3 = K, and the mode shape (Eigen vectors) are x1=123																					(13)

x1

x2

x3

x4

8.	Find the natural frequency of system shown in figure.

Use Stodala method only to find the answer.

(OR)

9.	Using Holzer method , determine the natural frequencies of the spring-mass system shown in figure.  Take M1 = M3 = 1 kg, M2 = 2kg, K1 = K2 = 1N/m.

X1

X2

X3

K1 =1

K2 =1

M1

M2

M3