Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	MEMORIES AND ANALOG SIGNAL CONDITIONING	       Time: 3 hours

Subject Code:	MT235		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is BiCMOS?

2.	An Astable multivibrator has — stable states.

3.	Define Q factor.

4.	What do you mean by stagger tuning?

5.	Draw the a.c equivalent of quartz crystal.

6.	The ripple factor of full-wave rectifier is _______.

7.	State the types of class A amplifier.

8.	_______ amplifiers are normally operated in a push-pull configuration in order to produce an output that is a replica of the input.

9.	Define SOA of MOSFET.

10.	Define transconductance.

PART – B (5 x 3 = 15 MARKS)

11.	What do mean by ECL? Draw the circuit diagram of ECL.  Also state its applications.

12.	Compare Butterworth and Chebyshev filters.

13.	State Barkhausen criterion for sustained oscillations. Classify Oscillators.

14.	Compare Class A and Class AB power amplifiers based on the following factors:

a.	conduction angle 		b.	position of Q-point 	c.	efficiency   	d.	distortion

15.	Discuss the various internal capacitances of Power MOSFET.

PART – C (5 x 15 = 75 MARKS)

16.	Explain, in detail, the different types of semiconductor memories.

(OR)

17.	a.	Explain the various types of Flip-flops with the help of their truth tables.			(8)

b.	Find the sense time of a Dynamic-latch sense amplifier.							(7)

18.	a.	Explain the principle of operation of single tuned amplifier.						(10)

b.	Discuss the effect Q on bandwidth of single tuned amplifier.						(5)

(OR)

19.	Derive the transfer function of a second order butterworth lowpass filter.

20.	Explain the principle of operation and derive the expression for frequency of oscillation of Wien Bridge Oscillator.

(OR)

21.	Explain the operation of centre tapped FWR with a neat circuit diagram and waveforms and derive Idc, Irms, Vdc, Vrms and efficiency.

22.	Explain and obtain the efficiency of transformer – coupled class A power amplifier.

(OR)

[P.T.O]

23.	Explain the principle of operation of class B push pull amplifier. Also, derive the expressions for the following,

a.	ICQ and VCEQ 	b.	AC power output		c.	DC power output   	d.	% Efficiency

24.	Discuss in detail about IC power amplifiers with one example.

(OR)

25.	a.	Draw and explain structure of n-channel power MOSFET. Also, discuss the various 	characteristics of power MOSFET.												(12)

b.	Draw the circuit diagram of Class AB power amplifier that uses MOSFET.			(3)