Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2012

Subject Title:	SIGNAL TRANSMISSION AND RADIATION			       Time: 3 hours

Subject Code:	MT238		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define propagation constant of a transmission line.

2.	Define VSWR.

3.	Define cut-off frequency of a wave-guide.

4.	What is the dominant mode for the TE waves in rectangular waveguide?

5.	The Critical frequency of E layer is ______.

6.	Define surface wave.

7.	What is an isotropic radiator?

8.	Define Power gain of an antenna.

9.	Name few splicing methods in fiber optics.

10.	Define fiber loss.

PART – B (5 x 3 = 15 MARKS)

11.	What are standing waves and SWR? State the relation between SWR and reflection co-efficient.

12.	What is a TEM wave? Mention its characteristics.

13.	What is Tropospheric wave propagation?

14.	An antenna has a radiation resistance of 72 Ohms, a loss resistance of 8 Ohms and a power gain of 12dB. Determine the antenna efficiency and its directivity.

15.	Compare LED and LASER.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the expression for the input impedance of a transmission line				(10)

b.	A Cable has the following parameters:

R: 48.75 ohm/km, L: 1.09 mH/km, G: 38.75 MU/km and C:0.59 f/km.  Determine the characteristics impedance, propagation constant and wavelength for a source of frequency 1600 Hz and Es: 1.0 volts.														(5)

(OR)

17.	a.	Derive the reflection loss of a transmission line.									(7)

b.	Explain how transmission line acts as circuit elements.							(8)

18.	Obtain the solution of Electric and Magnetic fields of TM waves guided along rectangular waveguide.

(OR)

19.	Explain the following:

a.	Radio-Frequency line		b.	Microstrip line									(8+7)

20.	Explain in detail about ionospheric wave propagation. Also explain the various layers of ionosphere.

(OR)

[P.T.O]

21.	Explain the following:

a.	LF propagation 			b.	VLF propagation 									(8+7)

22.	Derive the expressions for far field components of a small loop antenna.

(OR)

23.	Draw the structure of Yagi-uda Antenna. Explain the construction and operation of Yagi-uda antenna.

24.	Draw the structures of edge-emitting LED and surface emitting LED and explain the operation.

(OR)

25.	With neat block diagram, explain the various elements of fiber optic communication system.